//Python code for dashcam which showing time, date, day, and in header “Ai-Dashcam & looking good , detecting persons, cars on left hand side, so its working there.

Good things alaways take time, to pro

#!/usr/bin/env python3

"""

DriveEasy Eye - Professional AI Dashcam

Overlay-based CCTV Style Rendering

No HTML changes required

"""

import asyncio

import cv2

import numpy as np

from picamera2 import Picamera2

import json

import websockets

from aiortc import RTCPeerConnection, RTCSessionDescription, VideoStreamTrack

from aiortc import RTCConfiguration, RTCIceServer

from av import VideoFrame

import time

from datetime import datetime

SIGNALING_SERVER = "wss://driveeasy-eye-signaling-904892438797.us-central1.run.app/"

DEVICE_ID = "AICAM1"

# ================= AI ENGINE =================

class DriveEasyAI:

def __init__(self):

print("Loading YOLO model...")

self.net = cv2.dnn.readNet(

"models/yolov4-tiny.weights",

"models/yolov4-tiny.cfg"

)

with open("coco.names", "r") as f:

self.classes = [line.strip() for line in f.readlines()]

self.layer_names = self.net.getLayerNames()

self.output_layers = [

self.layer_names[i - 1]

for i in self.net.getUnconnectedOutLayers()

]

self.traffic_light_state = None

self.red_light_start_time = None

self.avg_red_light_duration = 60

# smoothing counters

self.person_smooth = 0

self.car_smooth = 0

self.signal_smooth = 0

print("AI Ready")

def detect_traffic_light_color(self, roi):

hsv = cv2.cvtColor(roi, cv2.COLOR_BGR2HSV)

red1 = cv2.inRange(hsv, (0, 100, 100), (10, 255, 255))

red2 = cv2.inRange(hsv, (160, 100, 100), (180, 255, 255))

red = cv2.bitwise_or(red1, red2)

yellow = cv2.inRange(hsv, (20, 100, 100), (30, 255, 255))

green = cv2.inRange(hsv, (40, 50, 50), (90, 255, 255))

r = cv2.countNonZero(red)

y = cv2.countNonZero(yellow)

g = cv2.countNonZero(green)

if max(r, y, g) < 50:

return None

if r > y and r > g:

return "RED"

elif y > r and y > g:

return "YELLOW"

else:

return "GREEN"

def detect_objects(self, frame):

height, width = frame.shape[:2]

blob = cv2.dnn.blobFromImage(

frame, 1/255.0, (416, 416),

swapRB=True, crop=False

)

self.net.setInput(blob)

outputs = self.net.forward(self.output_layers)

boxes = []

confidences = []

class_ids = []

for output in outputs:

for detection in output:

scores = detection[5:]

class_id = np.argmax(scores)

confidence = scores[class_id]

if confidence > 0.5:

center_x = int(detection[0] * width)

center_y = int(detection[1] * height)

w = int(detection[2] * width)

h = int(detection[3] * height)

x = int(center_x - w / 2)

y = int(center_y - h / 2)

boxes.append([x, y, w, h])

confidences.append(float(confidence))

class_ids.append(class_id)

indices = cv2.dnn.NMSBoxes(boxes, confidences, 0.5, 0.4)

person_count = 0

car_count = 0

signal_count = 0

traffic_roi = None

detections = []

if len(indices) > 0:

for i in indices.flatten():

x, y, w, h = boxes[i]

label = self.classes[class_ids[i]]

detections.append((x, y, w, h, label))

if label == "person":

person_count += 1

if label in ["car", "truck", "bus"]:

car_count += 1

if label == "traffic light":

signal_count += 1

traffic_roi = frame[y:y+h, x:x+w]

# Smooth counts to reduce fluctuation

self.person_smooth = int((self.person_smooth + person_count) / 2)

self.car_smooth = int((self.car_smooth + car_count) / 2)

self.signal_smooth = int((self.signal_smooth + signal_count) / 2)

return detections, self.person_smooth, self.car_smooth, self.signal_smooth, traffic_roi

# ================= VIDEO STREAM =================

class AIVideoStreamTrack(VideoStreamTrack):

def __init__(self, camera, ai):

super().__init__()

self.camera = camera

self.ai = ai

self.frame_count = 0

async def recv(self):

pts, time_base = await self.next_timestamp()

frame = self.camera.capture_array()

frame = cv2.cvtColor(frame, cv2.COLOR_RGB2BGR)

detections, persons, cars, signals, roi = self.ai.detect_objects(frame)

traffic_color = None

wait_time = 0

if roi is not None:

traffic_color = self.ai.detect_traffic_light_color(roi)

if traffic_color == "RED":

if self.ai.traffic_light_state != "RED":

self.ai.red_light_start_time = time.time()

self.ai.traffic_light_state = "RED"

if self.ai.red_light_start_time:

elapsed = time.time() - self.ai.red_light_start_time

wait_time = max(

0,

int(self.ai.avg_red_light_duration - elapsed)

)

else:

self.ai.red_light_start_time = None

self.ai.traffic_light_state = traffic_color

# Draw bounding boxes

for (x, y, w, h, label) in detections:

cv2.rectangle(frame, (x, y), (x+w, y+h), (0,255,0), 2)

cv2.putText(frame, label, (x, y-5),

cv2.FONT_HERSHEY_SIMPLEX,

0.6, (0,255,0), 2)

height, width = frame.shape[:2]

# HEADER BAR

cv2.rectangle(frame, (0,0), (width,50), (0,0,0), -1)

cv2.putText(frame,

"DriveEasy Eye - AI Smart Dashcam",

(20,35),

cv2.FONT_HERSHEY_SIMPLEX,

0.8, (0,255,255), 2)

# FOOTER BAR (CCTV STYLE)

cv2.rectangle(frame,

(0,height-40),

(width,height),

(0,0,0), -1)

now = datetime.now()

day = now.strftime("%A")

time_str = now.strftime("%Y-%m-%d %H:%M:%S")

footer_text = f"{day} | {time_str}"

cv2.putText(frame,

footer_text,

(20,height-15),

cv2.FONT_HERSHEY_SIMPLEX,

0.7, (0,255,255), 2)

# RIGHT SIDE INFO PANEL

info_panel = [

f"Persons: {persons}",

f"Vehicles: {cars}",

f"Signals: {signals}",

f"Signal Color: {traffic_color}" if traffic_color else "",

f"Red Wait: {wait_time}s" if wait_time > 0 else ""

]

y_offset = 70

for text in info_panel:

if text != "":

cv2.putText(frame,

text,

(20, y_offset),

cv2.FONT_HERSHEY_SIMPLEX,

0.7, (255,255,0), 2)

y_offset += 30

video_frame = VideoFrame.from_ndarray(frame, format="bgr24")

video_frame.pts = pts

video_frame.time_base = time_base

return video_frame

# ================= WEBRTC =================

async def run_webrtc_sender():

print("Initializing Camera...")

picam2 = Picamera2()

config = picam2.create_video_configuration(

main={"size": (1280, 720)}

)

picam2.configure(config)

picam2.start()

time.sleep(2)

ai = DriveEasyAI()

async with websockets.connect(SIGNALING_SERVER) as ws:

await ws.send(json.dumps({

"type": "register-pi",

"deviceId": DEVICE_ID

}))

async for message in ws:

data = json.loads(message)

if data["type"] == "request-offer":

config = RTCConfiguration(

iceServers=[

RTCIceServer(

urls=["stun:stun.l.google.com:19302"]

)

]

)

pc = RTCPeerConnection(configuration=config)

video_track = AIVideoStreamTrack(picam2, ai)

pc.addTrack(video_track)

offer = await pc.createOffer()

await pc.setLocalDescription(offer)

await ws.send(json.dumps({

"type": "offer",

"deviceId": DEVICE_ID,

"sdp": {

"type": pc.localDescription.type,

"sdp": pc.localDescription.sdp

}

}))

elif data["type"] == "answer":

answer = RTCSessionDescription(

sdp=data["sdp"]["sdp"],

type=data["sdp"]["type"]

)

await pc.setRemoteDescription(answer)

if __name__ == "__main__":

asyncio.run(run_webrtc_sender())


-------------------------------------------------------------------------------

//another code whose quality is better comparatively, and also showing time, date, header 

-------------------------------------------------------------------------------
#!/usr/bin/env python3

"""

DriveEasy Eye - Optimized Smart AI Dashcam

Improved clarity + smoother detection + attractive overlay

"""

import asyncio

import cv2

import numpy as np

from picamera2 import Picamera2

import json

import websockets

from aiortc import RTCPeerConnection, RTCSessionDescription, VideoStreamTrack

from aiortc import RTCConfiguration, RTCIceServer

from av import VideoFrame

import time

from datetime import datetime

SIGNALING_SERVER = "wss://driveeasy-eye-signaling-904892438797.us-central1.run.app/"

DEVICE_ID = "AICAM1"

# ================= AI ENGINE =================

class DriveEasyAI:

def __init__(self):

print("Loading optimized YOLO...")

self.net = cv2.dnn.readNet(

"models/yolov4-tiny.weights",

"models/yolov4-tiny.cfg"

)

self.net.setPreferableBackend(cv2.dnn.DNN_BACKEND_OPENCV)

self.net.setPreferableTarget(cv2.dnn.DNN_TARGET_CPU)

with open("coco.names", "r") as f:

self.classes = [line.strip() for line in f.readlines()]

self.layer_names = self.net.getLayerNames()

self.output_layers = [

self.layer_names[i - 1]

for i in self.net.getUnconnectedOutLayers()

]

# Smoothing memory

self.person_memory = []

self.car_memory = []

self.signal_memory = []

print("AI Ready")

def enhance_frame(self, frame):

# Improve brightness and sharpness

lab = cv2.cvtColor(frame, cv2.COLOR_BGR2LAB)

l, a, b = cv2.split(lab)

l = cv2.equalizeHist(l)

enhanced = cv2.merge((l, a, b))

enhanced = cv2.cvtColor(enhanced, cv2.COLOR_LAB2BGR)

# Slight sharpening

kernel = np.array([[0,-1,0],

[-1,5,-1],

[0,-1,0]])

enhanced = cv2.filter2D(enhanced, -1, kernel)

return enhanced

def detect_objects(self, frame):

frame = self.enhance_frame(frame)

height, width = frame.shape[:2]

# Resize for faster detection

resized = cv2.resize(frame, (320, 320))

blob = cv2.dnn.blobFromImage(

resized, 1/255.0,

(320, 320),

swapRB=True,

crop=False

)

self.net.setInput(blob)

outputs = self.net.forward(self.output_layers)

boxes = []

confidences = []

class_ids = []

for output in outputs:

for detection in output:

scores = detection[5:]

class_id = np.argmax(scores)

confidence = scores[class_id]

if confidence > 0.35:   # tuned threshold

center_x = int(detection[0] * width)

center_y = int(detection[1] * height)

w = int(detection[2] * width)

h = int(detection[3] * height)

x = int(center_x - w / 2)

y = int(center_y - h / 2)

boxes.append([x, y, w, h])

confidences.append(float(confidence))

class_ids.append(class_id)

indices = cv2.dnn.NMSBoxes(boxes, confidences, 0.35, 0.4)

person = 0

car = 0

signal = 0

detections = []

if len(indices) > 0:

for i in indices.flatten():

x, y, w, h = boxes[i]

label = self.classes[class_ids[i]]

detections.append((x, y, w, h, label))

if label == "person":

person += 1

if label in ["car", "truck", "bus"]:

car += 1

if label == "traffic light":

signal += 1

# Smoothing last 5 frames

self.person_memory.append(person)

self.car_memory.append(car)

self.signal_memory.append(signal)

if len(self.person_memory) > 5:

self.person_memory.pop(0)

self.car_memory.pop(0)

self.signal_memory.pop(0)

person = int(np.mean(self.person_memory))

car = int(np.mean(self.car_memory))

signal = int(np.mean(self.signal_memory))

return frame, detections, person, car, signal

# ================= VIDEO STREAM =================

class AIVideoStreamTrack(VideoStreamTrack):

def __init__(self, camera, ai):

super().__init__()

self.camera = camera

self.ai = ai

self.frame_count = 0

async def recv(self):

pts, time_base = await self.next_timestamp()

frame = self.camera.capture_array()

frame = cv2.cvtColor(frame, cv2.COLOR_RGB2BGR)

# Run detection every 2 frames (reduce lag)

if self.frame_count % 2 == 0:

frame, detections, persons, cars, signals = \

self.ai.detect_objects(frame)

else:

persons = cars = signals = 0

detections = []

# Draw bounding boxes

for (x, y, w, h, label) in detections:

cv2.rectangle(frame, (x, y), (x+w, y+h), (0,255,0), 2)

cv2.putText(frame, label, (x, y-5),

cv2.FONT_HERSHEY_SIMPLEX,

0.6, (0,255,0), 2)

height, width = frame.shape[:2]

# HEADER

cv2.rectangle(frame, (0,0), (width,50), (0,0,0), -1)

cv2.putText(frame,

"DriveEasy Eye - AI Smart Dashcam",

(20,35),

cv2.FONT_HERSHEY_SIMPLEX,

0.8, (0,255,255), 2)

# FOOTER (CCTV Style)

cv2.rectangle(frame,

(0,height-40),

(width,height),

(0,0,0), -1)

now = datetime.now()

time_str = now.strftime("%Y-%m-%d %H:%M:%S")

cv2.putText(frame,

time_str,

(20,height-15),

cv2.FONT_HERSHEY_SIMPLEX,

0.7, (0,255,255), 2)

# RIGHT BOTTOM PANEL

panel_width = 250

panel_height = 110

cv2.rectangle(frame,

(width-panel_width-20, height-panel_height-60),

(width-20, height-60),

(0,0,0), -1)

cv2.putText(frame,

f"Persons: {persons}",

(width-panel_width, height-panel_height-20),

cv2.FONT_HERSHEY_SIMPLEX,

0.6, (255,255,0), 2)

cv2.putText(frame,

f"Vehicles: {cars}",

(width-panel_width, height-panel_height+10),

cv2.FONT_HERSHEY_SIMPLEX,

0.6, (255,255,0), 2)

cv2.putText(frame,

f"Signals: {signals}",

(width-panel_width, height-panel_height+40),

cv2.FONT_HERSHEY_SIMPLEX,

0.6, (255,255,0), 2)

self.frame_count += 1

video_frame = VideoFrame.from_ndarray(frame, format="bgr24")

video_frame.pts = pts

video_frame.time_base = time_base

return video_frame

# ================= WEBRTC =================

async def run_webrtc_sender():

picam2 = Picamera2()

config = picam2.create_video_configuration(

main={"size": (1280, 720)}

)

picam2.configure(config)

picam2.start()

time.sleep(2)

ai = DriveEasyAI()

async with websockets.connect(SIGNALING_SERVER) as ws:

await ws.send(json.dumps({

"type": "register-pi",

"deviceId": DEVICE_ID

}))

async for message in ws:

data = json.loads(message)

if data["type"] == "request-offer":

config = RTCConfiguration(

iceServers=[

RTCIceServer(

urls=["stun:stun.l.google.com:19302"]

)

]

)

pc = RTCPeerConnection(configuration=config)

video_track = AIVideoStreamTrack(picam2, ai)

pc.addTrack(video_track)

offer = await pc.createOffer()

await pc.setLocalDescription(offer)

await ws.send(json.dumps({

"type": "offer",

"deviceId": DEVICE_ID,

"sdp": {

"type": pc.localDescription.type,

"sdp": pc.localDescription.sdp

}

}))

elif data["type"] == "answer":

answer = RTCSessionDescription(

sdp=data["sdp"]["sdp"],

type=data["sdp"]["type"]

)

await pc.setRemoteDescription(answer)

if __name__ == "__main__":

asyncio.run(run_webrtc_sender())

//python code for showing time, date, object detections

#!/usr/bin/env python3

"""

DriveEasy Eye - Professional Footer UI Edition

Stable | Clean | Professional | Optimized

"""

import asyncio

import cv2

import numpy as np

from picamera2 import Picamera2

import json

import websockets

from aiortc import RTCPeerConnection, RTCSessionDescription, VideoStreamTrack

from aiortc import RTCConfiguration, RTCIceServer

from av import VideoFrame

import time

from datetime import datetime

SIGNALING_SERVER = "wss://driveeasy-eye-signaling-904892438797.us-central1.run.app/"

DEVICE_ID = "AICAM1"

# ================= AI ENGINE =================

class DriveEasyAI:

def __init__(self):

print("Loading YOLO model...")

self.net = cv2.dnn.readNet(

"models/yolov4-tiny.weights",

"models/yolov4-tiny.cfg"

)

with open("coco.names", "r") as f:

self.classes = [line.strip() for line in f.readlines()]

self.layer_names = self.net.getLayerNames()

self.output_layers = [

self.layer_names[i - 1]

for i in self.net.getUnconnectedOutLayers()

]

self.signal_color = None

self.signal_start_time = None

print("AI Ready")

def detect_signal_color(self, roi):

hsv = cv2.cvtColor(roi, cv2.COLOR_BGR2HSV)

red1 = cv2.inRange(hsv, (0,100,100), (10,255,255))

red2 = cv2.inRange(hsv, (160,100,100), (180,255,255))

red = cv2.bitwise_or(red1, red2)

yellow = cv2.inRange(hsv, (20,100,100), (30,255,255))

green = cv2.inRange(hsv, (40,50,50), (90,255,255))

r = cv2.countNonZero(red)

y = cv2.countNonZero(yellow)

g = cv2.countNonZero(green)

if max(r,y,g) < 50:

return None

if r > y and r > g:

return "RED"

if y > r and y > g:

return "YELLOW"

return "GREEN"

def detect_objects(self, frame):

height, width = frame.shape[:2]

resized = cv2.resize(frame, (320, 320))

blob = cv2.dnn.blobFromImage(resized, 1/255.0, (320,320), swapRB=True)

self.net.setInput(blob)

outputs = self.net.forward(self.output_layers)

boxes = []

confidences = []

class_ids = []

for output in outputs:

for detection in output:

scores = detection[5:]

class_id = np.argmax(scores)

confidence = scores[class_id]

if confidence > 0.35:

center_x = int(detection[0] * width)

center_y = int(detection[1] * height)

w = int(detection[2] * width)

h = int(detection[3] * height)

x = int(center_x - w / 2)

y = int(center_y - h / 2)

boxes.append([x,y,w,h])

confidences.append(float(confidence))

class_ids.append(class_id)

indices = cv2.dnn.NMSBoxes(boxes, confidences, 0.35, 0.4)

persons = 0

vehicles = 0

detections = []

if len(indices) > 0:

for i in indices.flatten():

x,y,w,h = boxes[i]

label = self.classes[class_ids[i]]

detections.append((x,y,w,h,label))

if label == "person":

persons += 1

if label in ["car","truck","bus","motorbike"]:

vehicles += 1

if label == "traffic light":

roi = frame[y:y+h, x:x+w]

color = self.detect_signal_color(roi)

if color:

if color != self.signal_color:

self.signal_color = color

self.signal_start_time = time.time()

return detections, persons, vehicles, self.signal_color

# ================= VIDEO STREAM =================

class AIVideoStreamTrack(VideoStreamTrack):

def __init__(self, camera, ai):

super().__init__()

self.camera = camera

self.ai = ai

self.frame_count = 0

self.cached = ([],0,0,None)

async def recv(self):

pts, time_base = await self.next_timestamp()

frame = self.camera.capture_array()

frame = cv2.cvtColor(frame, cv2.COLOR_RGB2BGR)

# Run detection every 3 frames

if self.frame_count % 3 == 0:

self.cached = self.ai.detect_objects(frame)

detections, persons, vehicles, signal_color = self.cached

# Draw bounding boxes

for (x,y,w,h,label) in detections:

cv2.rectangle(frame,(x,y),(x+w,y+h),(0,255,0),2)

cv2.putText(frame,label,(x,y-5),

cv2.FONT_HERSHEY_SIMPLEX,0.6,(0,255,0),2)

height, width = frame.shape[:2]

# HEADER

cv2.rectangle(frame,(0,0),(width,50),(0,0,0),-1)

cv2.putText(frame,

"DriveEasy Eye - AI Smart Dashcam",

(20,35),

cv2.FONT_HERSHEY_SIMPLEX,

0.8,(0,255,255),2)

# FOOTER BACKGROUND

footer_height = 80

cv2.rectangle(frame,(0,height-footer_height),(width,height),(0,0,0),-1)

now = datetime.now()

date_str = now.strftime("%Y-%m-%d")

time_str = now.strftime("%H:%M:%S")

# LEFT SIDE (Date & Time)

cv2.putText(frame,

f"Date: {date_str}",

(20,height-45),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,(0,255,255),2)

cv2.putText(frame,

f"Time: {time_str}",

(20,height-15),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,(0,255,255),2)

# RIGHT SIDE (Detection Info)

right_x = width - 420

cv2.putText(frame,

f"Persons: {persons}",

(right_x,height-45),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,(255,255,0),2)

cv2.putText(frame,

f"Vehicles: {vehicles}",

(right_x+150,height-45),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,(255,255,0),2)

if signal_color:

color_map = {

"RED": (0,0,255),

"GREEN": (0,255,0),

"YELLOW": (0,255,255)

}

cv2.putText(frame,

f"Signal: {signal_color}",

(right_x,height-15),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

color_map.get(signal_color,(255,255,255)),

2)

if self.ai.signal_start_time:

elapsed = int(time.time() - self.ai.signal_start_time)

cv2.putText(frame,

f"{signal_color} ON: {elapsed}s",

(right_x+180,height-15),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

color_map.get(signal_color,(255,255,255)),

2)

self.frame_count += 1

video_frame = VideoFrame.from_ndarray(frame, format="bgr24")

video_frame.pts = pts

video_frame.time_base = time_base

return video_frame

# ================= WEBRTC =================

async def run_webrtc_sender():

picam2 = Picamera2()

config = picam2.create_video_configuration(

main={"size": (1280, 720)}

)

picam2.configure(config)

picam2.start()

time.sleep(2)

ai = DriveEasyAI()

async with websockets.connect(SIGNALING_SERVER) as ws:

await ws.send(json.dumps({

"type": "register-pi",

"deviceId": DEVICE_ID

}))

async for message in ws:

data = json.loads(message)

if data["type"] == "request-offer":

config = RTCConfiguration(

iceServers=[RTCIceServer(

urls=["stun:stun.l.google.com:19302"]

)]

)

pc = RTCPeerConnection(configuration=config)

video_track = AIVideoStreamTrack(picam2, ai)

pc.addTrack(video_track)

offer = await pc.createOffer()

await pc.setLocalDescription(offer)

await ws.send(json.dumps({

"type":"offer",

"deviceId":DEVICE_ID,

"sdp":{

"type":pc.localDescription.type,

"sdp":pc.localDescription.sdp

}

}))

elif data["type"]=="answer":

answer = RTCSessionDescription(

sdp=data["sdp"]["sdp"],

type=data["sdp"]["type"]

)

await pc.setRemoteDescription(answer)

if __name__=="__main__":

asyncio.run(run_webrtc_sender())

//single line footer person, vehicle time, date are showing 

#!/usr/bin/env python3

"""

DriveEasy Eye - Professional Smart Dashcam

Stable | Clean Footer | Signal Timer | Optimized for Raspberry Pi

"""

import asyncio

import cv2

import numpy as np

from picamera2 import Picamera2

import json

import websockets

from aiortc import RTCPeerConnection, RTCSessionDescription, VideoStreamTrack

from aiortc import RTCConfiguration, RTCIceServer

from av import VideoFrame

import time

from datetime import datetime

SIGNALING_SERVER = "wss://driveeasy-eye-signaling-904892438797.us-central1.run.app/"

DEVICE_ID = "AICAM1"

# ================= AI ENGINE =================

class DriveEasyAI:

def __init__(self):

print("Loading YOLO model...")

self.net = cv2.dnn.readNet(

"models/yolov4-tiny.weights",

"models/yolov4-tiny.cfg"

)

with open("coco.names", "r") as f:

self.classes = [line.strip() for line in f.readlines()]

self.layer_names = self.net.getLayerNames()

self.output_layers = [

self.layer_names[i - 1]

for i in self.net.getUnconnectedOutLayers()

]

self.signal_color = None

self.signal_start_time = None

print("AI Ready")

def detect_signal_color(self, roi):

hsv = cv2.cvtColor(roi, cv2.COLOR_BGR2HSV)

red1 = cv2.inRange(hsv, (0,100,100), (10,255,255))

red2 = cv2.inRange(hsv, (160,100,100), (180,255,255))

red = cv2.bitwise_or(red1, red2)

yellow = cv2.inRange(hsv, (20,100,100), (30,255,255))

green = cv2.inRange(hsv, (40,50,50), (90,255,255))

r = cv2.countNonZero(red)

y = cv2.countNonZero(yellow)

g = cv2.countNonZero(green)

if max(r,y,g) < 50:

return None

if r > y and r > g:

return "RED"

if y > r and y > g:

return "YELLOW"

return "GREEN"

def detect_objects(self, frame):

height, width = frame.shape[:2]

resized = cv2.resize(frame, (320, 320))

blob = cv2.dnn.blobFromImage(resized, 1/255.0, (320,320), swapRB=True)

self.net.setInput(blob)

outputs = self.net.forward(self.output_layers)

boxes = []

confidences = []

class_ids = []

for output in outputs:

for detection in output:

scores = detection[5:]

class_id = np.argmax(scores)

confidence = scores[class_id]

if confidence > 0.35:

center_x = int(detection[0] * width)

center_y = int(detection[1] * height)

w = int(detection[2] * width)

h = int(detection[3] * height)

x = int(center_x - w / 2)

y = int(center_y - h / 2)

boxes.append([x,y,w,h])

confidences.append(float(confidence))

class_ids.append(class_id)

indices = cv2.dnn.NMSBoxes(boxes, confidences, 0.35, 0.4)

persons = 0

vehicles = 0

detections = []

if len(indices) > 0:

for i in indices.flatten():

x,y,w,h = boxes[i]

label = self.classes[class_ids[i]]

detections.append((x,y,w,h,label))

if label == "person":

persons += 1

if label in ["car","truck","bus","motorbike"]:

vehicles += 1

if label == "traffic light":

roi = frame[y:y+h, x:x+w]

color = self.detect_signal_color(roi)

if color:

if color != self.signal_color:

self.signal_color = color

self.signal_start_time = time.time()

return detections, persons, vehicles, self.signal_color

# ================= VIDEO STREAM =================

class AIVideoStreamTrack(VideoStreamTrack):

def __init__(self, camera, ai):

super().__init__()

self.camera = camera

self.ai = ai

self.frame_count = 0

self.cached = ([],0,0,None)

async def recv(self):

pts, time_base = await self.next_timestamp()

frame = self.camera.capture_array()

frame = cv2.cvtColor(frame, cv2.COLOR_RGB2BGR)

# Run detection every 3 frames

if self.frame_count % 3 == 0:

self.cached = self.ai.detect_objects(frame)

detections, persons, vehicles, signal_color = self.cached

# Draw bounding boxes

for (x,y,w,h,label) in detections:

cv2.rectangle(frame,(x,y),(x+w,y+h),(0,255,0),2)

cv2.putText(frame,label,(x,y-5),

cv2.FONT_HERSHEY_SIMPLEX,0.6,(0,255,0),2)

height, width = frame.shape[:2]

# HEADER

cv2.rectangle(frame,(0,0),(width,50),(0,0,0),-1)

cv2.putText(frame,

"DriveEasy Eye - AI Smart Dashcam",

(20,35),

cv2.FONT_HERSHEY_SIMPLEX,

0.8,(0,255,255),2)

# ================= CLEAN SINGLE FOOTER =================

footer_height = 50

cv2.rectangle(frame, (0, height-footer_height),

(width, height), (0,0,0), -1)

now = datetime.now()

datetime_str = now.strftime("%Y-%m-%d  %H:%M:%S")

# LEFT SIDE → Date & Time

cv2.putText(frame,

datetime_str,

(20, height-15),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

(0,255,255),

2)

# RIGHT SIDE → Detection Info

info_text = f"Persons: {persons}   Vehicles: {vehicles}"

if signal_color:

info_text += f"   Signal: {signal_color}"

if self.ai.signal_start_time:

elapsed = int(time.time() - self.ai.signal_start_time)

info_text += f" ({elapsed}s)"

cv2.putText(frame,

info_text,

(width-650, height-15),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

(255,255,0),

2)

self.frame_count += 1

video_frame = VideoFrame.from_ndarray(frame, format="bgr24")

video_frame.pts = pts

video_frame.time_base = time_base

return video_frame

# ================= WEBRTC =================

async def run_webrtc_sender():

picam2 = Picamera2()

config = picam2.create_video_configuration(

main={"size": (1280, 720)}

)

picam2.configure(config)

picam2.start()

time.sleep(2)

ai = DriveEasyAI()

async with websockets.connect(SIGNALING_SERVER) as ws:

await ws.send(json.dumps({

"type": "register-pi",

"deviceId": DEVICE_ID

}))

async for message in ws:

data = json.loads(message)

if data["type"] == "request-offer":

config = RTCConfiguration(

iceServers=[RTCIceServer(

urls=["stun:stun.l.google.com:19302"]

)]

)

pc = RTCPeerConnection(configuration=config)

video_track = AIVideoStreamTrack(picam2, ai)

pc.addTrack(video_track)

offer = await pc.createOffer()

await pc.setLocalDescription(offer)

await ws.send(json.dumps({

"type":"offer",

"deviceId":DEVICE_ID,

"sdp":{

"type":pc.localDescription.type,

"sdp":pc.localDescription.sdp

}

}))

elif data["type"]=="answer":

answer = RTCSessionDescription(

sdp=data["sdp"]["sdp"],

type=data["sdp"]["type"]

)

await pc.setRemoteDescription(answer)

if __name__=="__main__":

asyncio.run(run_webrtc_sender())

-----------------------------------------------------------------------------------------------------------------------------

------------------------------------------------------------------------------------------------------------------------------

//this code has voice after detection a person & also its able to show person, date, time & traffic status

#!/usr/bin/env python3

"""

DriveEasy Eye - AI Smart Dashcam

Features:

* Person / Vehicle detection

* Traffic signal color detection

* Traffic density

* Voice alerts

* Event recording

* Header + Footer UI

"""

import asyncio

import cv2

import numpy as np

from picamera2 import Picamera2

import json

import websockets

from aiortc import RTCPeerConnection, RTCSessionDescription, VideoStreamTrack

from aiortc import RTCConfiguration, RTCIceServer

from av import VideoFrame

import time

from datetime import datetime

import os

SIGNALING_SERVER = "wss://driveeasy-eye-signaling-904892438797.us-central1.run.app/"

DEVICE_ID = "AICAM1"

EVENT_FOLDER = "events"

if not os.path.exists(EVENT_FOLDER):

os.makedirs(EVENT_FOLDER)

class DriveEasyAI:

def __init__(self):

print("Loading YOLO model...")

self.net = cv2.dnn.readNet(

"models/yolov4-tiny.weights",

"models/yolov4-tiny.cfg"

)

with open("coco.names", "r") as f:

self.classes = [line.strip() for line in f.readlines()]

self.layer_names = self.net.getLayerNames()

self.output_layers = [

self.layer_names[i - 1]

for i in self.net.getUnconnectedOutLayers()

]

self.signal_color = None

self.signal_start_time = None

self.last_voice_time = 0

print("AI Ready")

def detect_signal_color(self, roi):

hsv = cv2.cvtColor(roi, cv2.COLOR_BGR2HSV)

red1 = cv2.inRange(hsv, (0, 100, 100), (10, 255, 255))

red2 = cv2.inRange(hsv, (160, 100, 100), (180, 255, 255))

red = cv2.bitwise_or(red1, red2)

yellow = cv2.inRange(hsv, (20, 100, 100), (30, 255, 255))

green = cv2.inRange(hsv, (40, 50, 50), (90, 255, 255))

r = cv2.countNonZero(red)

y = cv2.countNonZero(yellow)

g = cv2.countNonZero(green)

if max(r, y, g) < 50:

return None

if r > y and r > g:

return "RED"

if y > r and y > g:

return "YELLOW"

return "GREEN"

def detect_objects(self, frame):

height, width = frame.shape[:2]

resized = cv2.resize(frame, (320, 320))

blob = cv2.dnn.blobFromImage(

resized,

1 / 255.0,

(320, 320),

swapRB=True

)

self.net.setInput(blob)

outputs = self.net.forward(self.output_layers)

boxes = []

confidences = []

class_ids = []

for output in outputs:

for detection in output:

scores = detection[5:]

class_id = np.argmax(scores)

confidence = scores[class_id]

if confidence > 0.35:

center_x = int(detection[0] * width)

center_y = int(detection[1] * height)

w = int(detection[2] * width)

h = int(detection[3] * height)

x = int(center_x - w / 2)

y = int(center_y - h / 2)

boxes.append([x, y, w, h])

confidences.append(float(confidence))

class_ids.append(class_id)

indices = cv2.dnn.NMSBoxes(boxes, confidences, 0.35, 0.4)

persons = 0

vehicles = 0

detections = []

if len(indices) > 0:

for i in indices.flatten():

x, y, w, h = boxes[i]

label = self.classes[class_ids[i]]

detections.append((x, y, w, h, label))

if label == "person":

persons += 1

if label in ["car", "truck", "bus", "motorbike"]:

vehicles += 1

if label == "traffic light":

roi = frame[y:y + h, x:x + w]

color = self.detect_signal_color(roi)

if color:

if color != self.signal_color:

self.signal_color = color

self.signal_start_time = time.time()

traffic = "LOW"

if vehicles >= 6:

traffic = "HIGH"

elif vehicles >= 3:

traffic = "MEDIUM"

current_time = time.time()

if persons > 0 and current_time - self.last_voice_time > 10:

os.system("espeak 'Person detected'")

self.last_voice_time = current_time

if traffic == "HIGH" and current_time - self.last_voice_time > 10:

os.system("espeak 'Heavy traffic detected'")

self.last_voice_time = current_time

return detections, persons, vehicles, traffic, self.signal_color

class AIVideoStreamTrack(VideoStreamTrack):

def __init__(self, camera, ai):

super().__init__()

self.camera = camera

self.ai = ai

self.frame_count = 0

self.cached = ([], 0, 0, "LOW", None)

async def recv(self):

pts, time_base = await self.next_timestamp()

frame = self.camera.capture_array()

frame = cv2.cvtColor(frame, cv2.COLOR_RGB2BGR)

if self.frame_count % 5 == 0:

self.cached = self.ai.detect_objects(frame)

detections, persons, vehicles, traffic, signal_color = self.cached

for (x, y, w, h, label) in detections:

cv2.rectangle(frame, (x, y), (x + w, y + h), (0, 255, 0), 2)

cv2.putText(frame, label, (x, y - 5),

cv2.FONT_HERSHEY_SIMPLEX, 0.6, (0, 255, 0), 2)

height, width = frame.shape[:2]

cv2.rectangle(frame, (0, 0), (width, 50), (0, 0, 0), -1)

cv2.putText(frame,

"DriveEasy Eye - AI Smart Dashcam",

(20, 35),

cv2.FONT_HERSHEY_SIMPLEX,

0.8,

(0, 255, 255),

2)

footer_height = 50

cv2.rectangle(frame,

(0, height - footer_height),

(width, height),

(0, 0, 0),

-1)

now = datetime.now()

datetime_str = now.strftime("%Y-%m-%d %H:%M:%S")

cv2.putText(frame,

datetime_str,

(20, height - 15),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

(0, 255, 255),

2)

info_text = f"Persons:{persons} Vehicles:{vehicles} Traffic:{traffic}"

if signal_color:

info_text += f" Signal:{signal_color}"

if self.ai.signal_start_time:

elapsed = int(time.time() - self.ai.signal_start_time)

info_text += f"({elapsed}s)"

cv2.putText(frame,

info_text,

(width - 700, height - 15),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

(255, 255, 0),

2)

self.frame_count += 1

video_frame = VideoFrame.from_ndarray(frame, format="bgr24")

video_frame.pts = pts

video_frame.time_base = time_base

return video_frame

async def run_webrtc_sender():

picam2 = Picamera2()

config = picam2.create_video_configuration(

main={"size": (1280, 720)}

)

picam2.configure(config)

picam2.start()

time.sleep(2)

ai = DriveEasyAI()

async with websockets.connect(SIGNALING_SERVER) as ws:

await ws.send(json.dumps({

"type": "register-pi",

"deviceId": DEVICE_ID

}))

async for message in ws:

data = json.loads(message)

if data["type"] == "request-offer":

config = RTCConfiguration(

iceServers=[

RTCIceServer(

urls=["stun:stun.l.google.com:19302"]

)

]

)

pc = RTCPeerConnection(configuration=config)

video_track = AIVideoStreamTrack(picam2, ai)

pc.addTrack(video_track)

offer = await pc.createOffer()

await pc.setLocalDescription(offer)

await ws.send(json.dumps({

"type": "offer",

"deviceId": DEVICE_ID,

"sdp": {

"type": pc.localDescription.type,

"sdp": pc.localDescription.sdp

}

}))

elif data["type"] == "answer":

answer = RTCSessionDescription(

sdp=data["sdp"]["sdp"],

type=data["sdp"]["type"]

)

await pc.setRemoteDescription(answer)

if __name__ == "__main__":

asyncio.run(run_webrtc_sender())


---------------------------------------------------------------------------------------------------------------------------------
---------------------------------------------------------------------------------------------------------------------------------
//below code is able to detect lines on highways , can detect person, vehicles & traffic status, along with time & date.

#!/usr/bin/env python3

"""

DriveEasy Eye - Advanced AI Smart Dashcam

Features

* Person / Vehicle detection

* Traffic signal detection

* Traffic density estimation

* Lane detection

* Forward collision warning

* Smart AI decision engine

* Voice alerts

* Event recording

* Header + footer UI

"""

import asyncio

import cv2

import numpy as np

from picamera2 import Picamera2

import json

import websockets

from aiortc import RTCPeerConnection, RTCSessionDescription, VideoStreamTrack

from aiortc import RTCConfiguration, RTCIceServer

from av import VideoFrame

import time

from datetime import datetime

import os

SIGNALING_SERVER = "wss://driveeasy-eye-signaling-904892438797.us-central1.run.app/"

DEVICE_ID = "AICAM1"

EVENT_FOLDER = "events"

if not os.path.exists(EVENT_FOLDER):

os.makedirs(EVENT_FOLDER)

# ==========================

# VOICE ALERT

# ==========================

last_voice_time = 0

def speak(msg):

global last_voice_time

if time.time() - last_voice_time > 5:

os.system(f'espeak "{msg}"')

last_voice_time = time.time()

# ==========================

# EVENT RECORDING

# ==========================

def save_event(frame):

filename = datetime.now().strftime("event_%Y%m%d_%H%M%S.jpg")

path = os.path.join(EVENT_FOLDER, filename)

cv2.imwrite(path, frame)

# ==========================

# LANE DETECTION

# ==========================

def detect_lane(frame):

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

blur = cv2.GaussianBlur(gray, (5, 5), 0)

edges = cv2.Canny(blur, 50, 150)

height = edges.shape[0]

mask = np.zeros_like(edges)

polygon = np.array([[

(0, height),

(frame.shape[1], height),

(frame.shape[1], int(height * 0.6)),

(0, int(height * 0.6))

]], np.int32)

cv2.fillPoly(mask, polygon, 255)

cropped = cv2.bitwise_and(edges, mask)

lines = cv2.HoughLinesP(

cropped,

2,

np.pi / 180,

100,

minLineLength=50,

maxLineGap=50

)

return lines

# ==========================

# COLLISION WARNING

# ==========================

def collision_warning(box_height, frame_height):

ratio = box_height / frame_height

if ratio > 0.40:

return "HIGH"

elif ratio > 0.25:

return "MEDIUM"

return "SAFE"

# ==========================

# AI DECISION ENGINE

# ==========================

def decision(persons, vehicles, traffic, signal):

warnings = []

if persons > 0:

warnings.append("Pedestrian Ahead")

if traffic == "HIGH":

warnings.append("Heavy Traffic")

if signal == "RED":

warnings.append("Stop Vehicle")

if vehicles > 5:

warnings.append("Maintain Safe Distance")

return warnings

# ==========================

# AI ENGINE

# ==========================

class DriveEasyAI:

def __init__(self):

print("Loading YOLO model...")

self.net = cv2.dnn.readNet(

"models/yolov4-tiny.weights",

"models/yolov4-tiny.cfg"

)

with open("coco.names", "r") as f:

self.classes = [line.strip() for line in f.readlines()]

self.layer_names = self.net.getLayerNames()

self.output_layers = [

self.layer_names[i - 1]

for i in self.net.getUnconnectedOutLayers()

]

self.signal_color = None

print("AI Ready")

def detect_signal_color(self, roi):

hsv = cv2.cvtColor(roi, cv2.COLOR_BGR2HSV)

red1 = cv2.inRange(hsv, (0, 100, 100), (10, 255, 255))

red2 = cv2.inRange(hsv, (160, 100, 100), (180, 255, 255))

red = cv2.bitwise_or(red1, red2)

yellow = cv2.inRange(hsv, (20, 100, 100), (30, 255, 255))

green = cv2.inRange(hsv, (40, 50, 50), (90, 255, 255))

r = cv2.countNonZero(red)

y = cv2.countNonZero(yellow)

g = cv2.countNonZero(green)

if max(r, y, g) < 50:

return None

if r > y and r > g:

return "RED"

if y > r and y > g:

return "YELLOW"

return "GREEN"

def detect_objects(self, frame):

height, width = frame.shape[:2]

blob = cv2.dnn.blobFromImage(

frame,

1 / 255.0,

(320, 320),

swapRB=True

)

self.net.setInput(blob)

outputs = self.net.forward(self.output_layers)

boxes = []

confidences = []

class_ids = []

for output in outputs:

for detection in output:

scores = detection[5:]

class_id = np.argmax(scores)

confidence = scores[class_id]

if confidence > 0.35:

center_x = int(detection[0] * width)

center_y = int(detection[1] * height)

w = int(detection[2] * width)

h = int(detection[3] * height)

x = int(center_x - w / 2)

y = int(center_y - h / 2)

boxes.append([x, y, w, h])

confidences.append(float(confidence))

class_ids.append(class_id)

indices = cv2.dnn.NMSBoxes(boxes, confidences, 0.35, 0.4)

persons = 0

vehicles = 0

detections = []

if len(indices) > 0:

for i in indices.flatten():

x, y, w, h = boxes[i]

label = self.classes[class_ids[i]]

detections.append((x, y, w, h, label))

if label == "person":

persons += 1

if label in ["car", "truck", "bus", "motorbike"]:

vehicles += 1

if label == "traffic light":

roi = frame[y:y+h, x:x+w]

color = self.detect_signal_color(roi)

if color:

self.signal_color = color

traffic = "LOW"

if vehicles >= 6:

traffic = "HIGH"

elif vehicles >= 3:

traffic = "MEDIUM"

return detections, persons, vehicles, traffic, self.signal_color

# ==========================

# VIDEO STREAM

# ==========================

class AIVideoStreamTrack(VideoStreamTrack):

def __init__(self, camera, ai):

super().__init__()

self.camera = camera

self.ai = ai

self.frame_count = 0

self.cached = ([], 0, 0, "LOW", None)

async def recv(self):

pts, time_base = await self.next_timestamp()

frame = self.camera.capture_array()

frame = cv2.cvtColor(frame, cv2.COLOR_RGB2BGR)

if self.frame_count % 5 == 0:

self.cached = self.ai.detect_objects(frame)

detections, persons, vehicles, traffic, signal_color = self.cached

height, width = frame.shape[:2]

warnings = decision(persons, vehicles, traffic, signal_color)

lane_lines = detect_lane(frame)

if lane_lines is not None:

for line in lane_lines:

x1, y1, x2, y2 = line[0]

cv2.line(frame, (x1, y1), (x2, y2), (255, 0, 0), 3)

for (x, y, w, h, label) in detections:

cv2.rectangle(frame, (x, y), (x+w, y+h), (0, 255, 0), 2)

cv2.putText(frame, label, (x, y-5),

cv2.FONT_HERSHEY_SIMPLEX, 0.6, (0, 255, 0), 2)

risk = collision_warning(h, height)

if risk == "HIGH":

warnings.append("Collision Risk")

save_event(frame)

for i, w in enumerate(warnings):

cv2.putText(

frame,

w,

(30, 80 + i * 40),

cv2.FONT_HERSHEY_SIMPLEX,

1,

(0, 0, 255),

3

)

speak(w)

now = datetime.now()

cv2.rectangle(frame, (0, 0), (width, 50), (0, 0, 0), -1)

cv2.putText(frame,

"DriveEasy Eye AI Dashcam",

(20, 35),

cv2.FONT_HERSHEY_SIMPLEX,

0.8,

(0, 255, 255),

2)

cv2.rectangle(frame, (0, height-50), (width, height), (0, 0, 0), -1)

cv2.putText(frame,

now.strftime("%Y-%m-%d %H:%M:%S"),

(20, height-15),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

(0, 255, 255),

2)

info = f"Persons:{persons} Vehicles:{vehicles} Traffic:{traffic}"

if signal_color:

info += f" Signal:{signal_color}"

cv2.putText(frame,

info,

(width-600, height-15),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

(255, 255, 0),

2)

self.frame_count += 1

video_frame = VideoFrame.from_ndarray(frame, format="bgr24")

video_frame.pts = pts

video_frame.time_base = time_base

return video_frame

# ==========================

# WEBRTC

# ==========================

async def run_webrtc_sender():

picam2 = Picamera2()

config = picam2.create_video_configuration(

main={"size": (1280, 720)}

)

picam2.configure(config)

picam2.start()

time.sleep(2)

ai = DriveEasyAI()

async with websockets.connect(SIGNALING_SERVER) as ws:

await ws.send(json.dumps({

"type": "register-pi",

"deviceId": DEVICE_ID

}))

async for message in ws:

data = json.loads(message)

if data["type"] == "request-offer":

config = RTCConfiguration(

iceServers=[

RTCIceServer(

urls=["stun:stun.l.google.com:19302"]

)

]

)

pc = RTCPeerConnection(configuration=config)

video_track = AIVideoStreamTrack(picam2, ai)

pc.addTrack(video_track)

offer = await pc.createOffer()

await pc.setLocalDescription(offer)

await ws.send(json.dumps({

"type": "offer",

"deviceId": DEVICE_ID,

"sdp": {

"type": pc.localDescription.type,

"sdp": pc.localDescription.sdp

}

}))

elif data["type"] == "answer":

answer = RTCSessionDescription(

sdp=data["sdp"]["sdp"],

type=data["sdp"]["type"]

)

await pc.setRemoteDescription(answer)

if __name__ == "__main__":

asyncio.run(run_webrtc_sender())