# DriveEasy AI Dashcam Deployment Report

System: Raspberry Pi-Based AI Dashcam
Version: Deployment v1.0

## 1. Objective

This document provides a complete, professional deployment guide for setting up the DriveEasy AI Dashcam system on a new Raspberry Pi. The goal is to ensure:

- Smooth, lag-free video streaming

- Reliable camera operation

- Automatic startup on boot

- Scalable architecture for future AI integration

## 2. System Requirements

### Hardware

- Raspberry Pi (Pi 3 / Pi 4 / Pi 5 recommended)

- Raspberry Pi Camera Module (CSI) or USB webcam

- MicroSD Card (16GB+ recommended)

- Stable power supply

- Internet connectivity (WiFi or Ethernet)

### Software

- Raspberry Pi OS (latest version)

- Python 3.9+

- Required Python libraries (aiortc, OpenCV, etc.)

## 3. Operating System Installation

- Install Raspberry Pi OS using Raspberry Pi Imager.

- Enable the following during setup:

  - SSH access

  - WiFi configuration

- Boot the Raspberry Pi and log in.

## 4. System Initialization

Update the system packages:

sudo apt update

sudo apt upgrade -y

## 5. Camera Configuration

Enable camera interface:

sudo raspi-config

Navigate to:

Interface Options → Camera → Enable

Reboot the system:

sudo reboot

Verify camera functionality:

libcamera-hello

## 6. Dependency Installation

Install required system packages:

sudo apt install -y python3-pip python3-venv python3-opencv libcamera-apps

## 7. Project Setup

Create project directory:

mkdir ~/ai_dashcam

cd ~/ai_dashcam

## 8. Python Virtual Environment

Create and activate virtual environment:

python3 -m venv venv

source venv/bin/activate

Install required Python libraries:

pip install aiortc websockets av numpy opencv-python

## 9. Application Deployment

Create the main application file:

nano webrtc_ai_sender.py

Paste the working streaming code into this file.

## 10. Manual Testing

Run the application:

python webrtc_ai_sender.py

Verify:

- Camera initializes successfully

- No runtime errors

- Video stream appears on UI

## 11. Camera Resource Handling

Prevent camera lock issues:

sudo pkill -f libcamera

This ensures no process blocks camera access.

## 12. Auto-Start Service Configuration

Create systemd service:

sudo nano /etc/systemd/system/driveeasy.service

Add configuration:

[Unit]

Description=DriveEasy AI Dashcam

After=network-online.target

Wants=network-online.target

[Service]

User=pi

WorkingDirectory=/home/pi/ai_dashcam

ExecStartPre=/bin/sleep 5

ExecStartPre=-/usr/bin/pkill -f libcamera

ExecStart=/home/pi/ai_dashcam/venv/bin/python /home/pi/ai_dashcam/webrtc_ai_sender.py

Restart=always

RestartSec=5

StandardOutput=journal

StandardError=journal

[Install]

WantedBy=multi-user.target

## 13. Enable and Start Service

sudo systemctl daemon-reload

sudo systemctl enable driveeasy.service

sudo systemctl start driveeasy.service

## 14. Verification

Check service status:

systemctl status driveeasy.service

Expected output:

active (running)

## 15. Boot Validation

Reboot system:

sudo reboot

After reboot:

systemctl status driveeasy.service

Confirm automatic startup.

## 16. Supported Camera Modules

- Raspberry Pi Camera Module v2 (IMX219)

- Raspberry Pi Camera Module v3 (IMX708)

- USB webcams (UVC compatible)

## 17. Common Issues & Solutions

| Issue | Cause | Solution |
| --- | --- | --- |
| Camera busy | Multiple processes | sudo pkill -f libcamera |
| Module not found | venv not active | Activate venv |
| Service fails | Path/config error | Check journal logs |
| No video stream | WebRTC issue | Verify signaling |

## 18. Performance Optimization Notes

- Use lower resolution (640×480) for smoother performance

- Avoid heavy AI models during baseline testing

- Use frame skipping for AI workloads

- Separate AI processing and streaming (future enhancement)

## 19. Future Enhancements

- AI-based object detection (YOLO / TensorFlow Lite)

- GPS integration and mapping

- Event-based recording system

- Gemini AI assistant integration

- Multi-threaded processing pipeline

## 20. Conclusion

The DriveEasy AI Dashcam system has been successfully structured for:

- Reliable deployment

- Smooth video streaming

- Scalable architecture

- Production-ready service execution

This setup ensures a plug-and-play experience for any new Raspberry Pi device with minimal configuration effort.