# AI Dashcam Setup Guide (64 GB SD Card)

This document provides a complete step-by-step guide to set up the AI Dashcam system on a Raspberry Pi using a 64 GB SD card.

It supports YOLOv8 object detection, WebRTC live streaming, and video recording.

==================================================

SYSTEM REQUIREMENTS

==================================================

• Raspberry Pi 4 or Pi 5

• 64 GB SD Card (expanded filesystem)

• Raspberry Pi Camera / USB Camera

• Internet connection (Wi-Fi / Hotspot / GSM)

• Raspberry Pi OS (Bookworm / Trixie)

==================================================

PROJECT FOLDER STRUCTURE

==================================================

/home/driveeasy/ai_dashcam/

├── main.py

├── detector.py

├── overlay.py

├── traffic_light.py

├── recorder.py

├── video_source.py

├── webrtc_ai_sender.py

├── requirements.txt

├── models/

│   └── yolov8n.pt

└── recordings/

==================================================

STEP 1: CLEAN SYSTEM

==================================================

cd ~

rm -rf ~/.cache/pip

sudo apt clean

sudo apt autoremove -y

sudo journalctl --vacuum-size=200M

==================================================

STEP 2: VERIFY STORAGE

==================================================

df -h

If filesystem is not expanded:

sudo raspi-config

→ Advanced Options → Expand Filesystem → Reboot

==================================================

STEP 3: CHECK INTERNET

==================================================

ping -c 3 8.8.8.8

ping -c 3 github.com

==================================================

STEP 4: INSTALL SYSTEM PACKAGES

==================================================

sudo apt update

sudo apt install -y python3-full python3-venv python3-opencv python3-pip git ffmpeg

==================================================

STEP 5: CREATE VIRTUAL ENVIRONMENT

==================================================

cd ~/ai_dashcam

python3 -m venv venv

source venv/bin/activate

==================================================

STEP 6: INSTALL PYTHON LIBRARIES

==================================================

pip install --upgrade pip wheel setuptools

pip install numpy python-dateutil

pip install ultralytics

==================================================

STEP 7: DOWNLOAD YOLOv8 MODEL

==================================================

mkdir -p models

cd models

wget https://github.com/ultralytics/assets/releases/download/v8.0.0/yolov8n.pt

cd ..

==================================================

STEP 8: TEST YOLO MODEL

==================================================

from ultralytics import YOLO

YOLO("models/yolov8n.pt")

==================================================

STEP 9: CHECK CAMERA

==================================================

ls /dev/video*

==================================================

STEP 10: RUN AI DASHCAM

==================================================

cd ~/ai_dashcam

source venv/bin/activate

python main.py

==================================================

OUTPUT

==================================================

• Live WebRTC streaming

• Vehicle and traffic signal detection

• Signal timer overlay

• Date & time overlay

• Recorded MP4 videos

==================================================

IMPORTANT NOTES

==================================================

• Always activate virtual environment before running

• 64 GB SD card is recommended

• Do not install YOLOv4-tiny for this setup