# DriveEasy GSM Integration Guide

A7670C LTE Module Integration with Raspberry Pi 4

## 1. Introduction

This document explains the complete hardware and software integration process for connecting the A7670C LTE GSM module with Raspberry Pi 4. It also clarifies limitations related to UART-only modules and streaming applications.

## 2. Important Architecture Clarification

The A7670C module without a USB interface exposes only UART (TX/RX) communication. UART supports AT commands and basic TCP socket communication but is NOT suitable for high-bandwidth applications such as WebRTC video streaming.

UART Maximum Practical Throughput:

- 115200 baud ≈ 11 KB/s

- 921600 baud ≈ 90 KB/s

WebRTC streaming requires 300–800 KB/s minimum.

Conclusion: UART-only A7670C cannot support real-time video streaming.

## 3. Hardware Connection (UART Mode)

If using UART interface, connect as follows:

A7670C TXD  → Raspberry Pi RX (GPIO15, Pin 10)

A7670C RXD  → Raspberry Pi TX (GPIO14, Pin 8)

A7670C GND  → Raspberry Pi GND (Pin 6)

A7670C VCC  → External 4.0V regulated supply

IMPORTANT: The A7670C requires 3.8V–4.2V with peak current up to 2A. Do NOT power the module directly from Raspberry Pi 5V pin. Use a buck converter (5V → 4V) with at least 1000µF capacitor near VBAT.

## 4. Enable UART on Raspberry Pi

Run:

sudo raspi-config

Navigate to: Interface Options → Serial Port

Disable login shell over serial

Enable serial hardware

Reboot the system

## 5. Test AT Communication

Install minicom:

sudo apt install minicom

Run:

sudo minicom -b 115200 -o -D /dev/serial0

Type AT and expect response OK.

## 6. LTE Data Connection Using AT Commands

Basic command sequence for Airtel IoT SIM:

AT+CPIN?

AT+CSQ

AT+CREG?

AT+CGATT=1

AT+CGDCONT=1,"IP","airtelgprs.com"

AT+NETOPEN

AT+IPADDR

Note: This creates a TCP/IP session inside the module, but does not create a Linux network interface (like wwan0).

## 7. Recommended Production Architecture

For real-time AI dashcam streaming using WebRTC, the recommended solution is:

- A7670C USB variant OR

- SIM7600 USB LTE modem OR

- Standard 4G USB LTE dongle

These devices create a Linux network interface (wwan0), allowing Raspberry Pi to use LTE as a normal internet connection.

## 8. Final Recommendation for DriveEasy System

For a production-grade AI dashcam system requiring cloud streaming, WiFi + USB LTE modem architecture is strongly recommended over UART-only GSM modules.

This ensures:

- Stable Internet Interface

- Full WebRTC compatibility

- Higher upload bandwidth

- Reduced system complexity

End of Document