# DriveEasy Eye - Stable Raspberry Pi AI Dashcam Setup Guide

This document provides a complete step-by-step stable setup for Raspberry Pi 4 AI Dashcam with WebRTC streaming and Picamera2 support.

Recommended Environment:

- Raspberry Pi OS Bookworm (64-bit)

- Python 3.11 (Stable Version)

- Picamera2 via apt (NOT pip)

- YOLOv4-tiny with OpenCV DNN

## STEP 1: Verify Operating System

Run the following command:

cat /etc/os-release

Ensure it shows VERSION="12 (bookworm)"

## STEP 2: Check Python Version

Run:

python3 --version

Ensure version is Python 3.11.x

If Python 3.13 is installed, downgrade to Python 3.11 for stability.

## STEP 3: Update System

Run:

sudo apt update

sudo apt full-upgrade -y

## STEP 4: Install Stable Camera Stack

Install official camera packages:

sudo apt install -y python3-picamera2 libcamera-apps python3-opencv python3-av python3-numpy

Do NOT install picamera2 using pip.

## STEP 5: Test Camera

Test libcamera preview:

libcamera-hello

Test Picamera2 in Python:

python3

from picamera2 import Picamera2

picam2 = Picamera2()

picam2.start()

print('Camera working stable')

## STEP 6: Create Virtual Environment for WebRTC

Navigate to project folder:

cd ~/ai_dashcam

Create virtual environment:

python3 -m venv venv

Activate it:

source venv/bin/activate

## STEP 7: Install WebRTC Dependencies

Inside virtual environment install:

pip install aiortc==1.5.0

pip install websockets==11.0.3

## STEP 8: Run AI Dashcam Sender

Activate venv and run:

cd ~/ai_dashcam

source venv/bin/activate

python webrtc_ai_sender.py

## Stable Working Stack Summary

- OS: Raspberry Pi OS Bookworm 64-bit

- Python: 3.11

- Picamera2: apt version

- OpenCV: apt version

- aiortc: 1.5.0

- websockets: 11.0.3

- YOLO: yolov4-tiny (OpenCV DNN)

This configuration ensures stable camera initialization, WebRTC streaming, and AI detection without allocator or compatibility errors.