# DriveEasy Eye - Complete ADAS System Documentation

This document provides a complete technical guide for building the DriveEasy Eye AI Dashcam system using Raspberry Pi 4, YOLOv4-tiny (OpenCV DNN), Bosch + BDD100K datasets, Norfair tracking, distance estimation, traffic signal detection, timer detection, and WebRTC streaming.

## 1. System Overview

Hardware: Raspberry Pi 4 + Pi Camera
Software: Python 3.x + OpenCV DNN
Model: YOLOv4-tiny (Custom trained)
Tracking: Norfair
Timer Detection: Tesseract OCR
Streaming: WebRTC (aiortc)

## 2. Raspberry Pi Environment Setup

Install system packages:

sudo apt update

sudo apt install python3-opencv python3-numpy -y

sudo apt install python3-picamera2 -y

sudo apt install tesseract-ocr -y

Create virtual environment:

python3 -m venv venv --system-site-packages

source venv/bin/activate

Install Python libraries:

pip install aiortc websockets av norfair pytesseract

## 3. Project Folder Structure

driveeasy_eye/
├── venv/
├── models/
│   ├── yolov4-tiny.cfg
│   ├── yolov4-tiny.weights
│   ├── coco.names
├── ai/
│   ├── detector.py
│   ├── tracker.py
│   ├── distance.py
│   ├── traffic_light.py
│   └── timer.py
├── webrtc/
├── config.py
└── main.py

## 4. Dataset Preparation & Training (Google Colab)

Datasets Used:
- BDD100K (Vehicles)
- Bosch Small Traffic Lights (Signal Colors)
- Custom Timer Dataset

Final Classes:
0 car
1 truck
2 bus
3 traffic_light_red
4 traffic_light_green
5 traffic_light_yellow
6 traffic_timer

Modify YOLOv4-tiny config:
classes=7
filters=36 (in both YOLO layers)

Train using Darknet in Colab:
./darknet detector train data/obj.data yolov4-tiny-custom.cfg yolov4-tiny.conv.29 -map

## 5. Distance Estimation Formula

Distance = (Real_Height × Focal_Length) / Bounding_Box_Height
Use smoothing for stability:
distance = 0.7 * previous_distance + 0.3 * new_distance

## 6. Norfair Tracking Integration

Install Norfair: pip install norfair
Initialize tracker:
tracker = Tracker(distance_function='euclidean', distance_threshold=30)
Convert YOLO detections to Norfair Detection objects
Update tracker each frame and draw object IDs

## 7. Traffic Timer Detection (OCR)

Use Tesseract OCR for digit extraction:
gray = cv2.cvtColor(timer_roi, cv2.COLOR_BGR2GRAY)
_, thresh = cv2.threshold(gray, 150, 255, cv2.THRESH_BINARY)
timer_text = pytesseract.image_to_string(thresh, config='--psm 7 -c tessedit_char_whitelist=0123456789')

## 8. Performance Optimization for Raspberry Pi 4

Use 640x480 resolution
Run detection every 3rd frame
Keep classes <= 7
Use YOLOv4-tiny only
Expected FPS: 12-18

## 9. Final Capabilities

Vehicle detection
Traffic signal color detection
Traffic timer detection
Distance estimation
Collision warning
Red-light violation detection
Object tracking
Live WebRTC streaming