EC200U-CN MODULE

CORRECTED COMPLETE CONNECTION REPORT

Schematic: EC200UCNAA-N05-SGNSA  |  KiCad 9.0.6  |  All Pins  |  14 Errors Fixed

✘  CRITICAL ERROR:  5 CRITICAL ERRORS FOUND IN YOUR SCHEMATIC. Fix all red items before ordering PCBs.

# Section 1 — All Errors Found in Your EC200U Schematic

The following errors were identified by comparing your uploaded schematic (EC200UCNAA-N05-SGNSA symbol) against the Quectel EC200U Series Hardware Design specification. Red = Critical (device won't work or will be damaged). Orange = High (instability or missing connection).

| # | EC200U Pin | Current Wrong Connection | Correct Connection | Severity |
| --- | --- | --- | --- | --- |
| 1 | Pin 64 — MAIN_DTR | UART2_TX net connected here | UART2_TX must go to MAIN_RXD (pin 62). DTR is a modem control line, NOT data! | 🔴 CRITICAL |
| 2 | Pin 62 — MAIN_RXD | UART2_RX net connected here | UART2_RX must come FROM MAIN_TXD (pin 63). MCU_RX ← Module_TX is correct direction. | 🔴 CRITICAL |
| 3 | Pin 63 — MAIN_TXD | No connection (NC) — floating! | Must connect to UART2_RX (ESP32 IO16). Module transmits → ESP32 receives. | 🔴 CRITICAL |
| 4 | Pin 50 — VBAT_BB | Connected to VCC_5V (5.0V) | Must be 3.4V–4.3V ONLY. Add 4.2V LDO (AMS1117-4.2) between 5V and VBAT_BB. | 🔴 CRITICAL |
| 5 | Pin 57 — VBAT_RF | Connected to VCC_5V (5.0V) | Must be 3.4V–4.3V. Connect same 4.2V rail as VBAT_BB. VBAT_RF = PA power. | 🔴 CRITICAL |
| 6 | Pin 128 — VDD_EXT | Connected to VCC_1V8 rail | VDD_EXT is an OUTPUT from EC200U (module provides 1.8V). Do NOT connect to LDO output. Use VDD_EXT to power other 1.8V peripherals, or leave NC. | 🟠 HIGH |
| 7 | Pin 64 — MAIN_DTR | UART2_TX incorrectly here | MAIN_DTR should connect to ESP32 IO33 (DTR signal for sleep control). Leave NC if sleep not needed. | 🟠 HIGH |
| 8 | Pin 61 — MAIN_RTS | Floating / NC | Tie to GND (3.3V logic) to disable hardware flow control, OR connect to ESP32 GPIO if HW flow control is used. | 🟡 MEDIUM |
| 9 | Pin 19 — STATUS | Net label: NMODULE_STATUS | In previous report label was MODULE_STATUS. Rename net in KiCad to MODULE_STATUS and connect to ESP32 IO35 (input). HIGH = module ON. | 🟡 MEDIUM |
| 10 | Pin 23 — SIM_DET | NC / floating | Connect to SIM card holder card-detect switch pin. Add 47kΩ pull-up to VDD_EXT (1.8V). LOW = SIM inserted. | 🟡 MEDIUM |
| 11 | GRF_C1 / GRF_C2 | No 100nF caps visible | REQUIRED: Add 100nF capacitor from each GRF_C1 (pin 143) and GRF_C2 (pin 139) to GND. Quectel hardware design mandatory. | 🟠 HIGH |
| 12 | Pin 1 — WAKEUP_IN | NC / floating | Pull HIGH to VDD_EXT (1.8V) via 47kΩ to keep module awake. Or connect to ESP32 GPIO for controlled wake. | 🟡 MEDIUM |
| 13 | Pin 47 — ANT_GNSS | No SMA connection visible on this pin | Must connect to J3 SMA_Edge_SMD via 50Ω microstrip trace. This is the GNSS antenna port. | 🟠 HIGH |
| 14 | Pin 48 — ANT_MAIN | No SMA connection visible on this pin | Must connect to J2 SMA_Edge_SMD via 50Ω microstrip trace. This is the LTE antenna port. | 🟠 HIGH |

✘  CRITICAL ERROR:  The 3 UART errors (#1, #2, #3) mean: EC200U will NEVER respond to AT commands, GPS data will NEVER arrive. The design will appear completely dead even if power is correct.

✘  CRITICAL ERROR:  VBAT errors (#4, #5): Powering EC200U at 5V will destroy the module permanently. EC200U VBAT max = 4.3V absolute maximum.

⚠  WARNING:  VDD_EXT error (#6): Connecting two voltage sources together (LDO output + module output) on the same net will cause one to fight the other — resulting in excess current, heating, and possible damage.

# Section 2 — UART Connection Correction (Most Important Fix)

## 2.1 What Is Wrong in Your Schematic:

- UART2_TX net is connected to EC200U pin 64 (MAIN_DTR). MAIN_DTR is a modem handshaking control line (Data Terminal Ready), NOT a data pin. Your ESP32 AT commands go nowhere.

- UART2_RX net is connected to EC200U pin 62 (MAIN_RXD). MAIN_RXD is the pin where EC200U RECEIVES data from you — not transmits. So ESP32 RX is listening to the wrong line.

- EC200U pin 63 (MAIN_TXD) — the pin that actually sends AT responses back — has NO CONNECTION at all. This is why the module will seem completely silent.

## 2.2 Correct UART Connections — Full Table:

| UART Channel | ESP32 Pin | ESP32 Function | Wire Direction | EC200U Pin # | EC200U Signal | Baud Rate | Net Label |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UART0 (Debug) | IO1 (TXD0) | TX — ESP32 transmits | ESP32 → CP2102N RXD | — | CP2102N RXD | 115200 | DEBUG_TX |
| UART0 (Debug) | IO3 (RXD0) | RX — ESP32 receives | CP2102N TXD → ESP32 | — | CP2102N TXD | 115200 | DEBUG_RX |
| UART2 (EC200U MAIN) — CORRECTED | IO17 | TX — ESP32 transmits | ESP32 IO17 → EC200U pin 62 (MAIN_RXD) | Pin 62 — MAIN_RXD | MAIN_RXD (module receives) | 115200 | UART2_TX |
| UART2 (EC200U MAIN) — CORRECTED | IO16 | RX — ESP32 receives | EC200U pin 63 (MAIN_TXD) → ESP32 IO16 | Pin 63 — MAIN_TXD | MAIN_TXD (module transmits) | 115200 | UART2_RX |
| DTR (was wrongly UART2_TX) | IO33 | OUT — sleep control | ESP32 IO33 → EC200U pin 64 (MAIN_DTR) | Pin 64 — MAIN_DTR | DTR sleep control signal | — | DTR |
| UART1 (EC200U GNSS) | IO13 | RX — ESP32 receives | EC200U GNSS_TXD → ESP32 IO13 | GNSS_TXD | GNSS transmits NMEA | 9600 | GNSS_DATA |
| UART1 (EC200U GNSS) | IO12 | TX — ESP32 transmits | ESP32 IO12 → EC200U GNSS_RXD | GNSS_RXD | GNSS receives config | 9600 | GNSS_CMD |

✔  Golden rule: MCU_TX → Module_RXD (module receives your commands). MCU_RX ← Module_TXD (module sends back responses). TX→RX, RX←TX — always cross-connect.

## 2.3 Step-by-Step Fix in KiCad:

- Delete the UART2_TX net label from pin 64 (MAIN_DTR)

- Add net label UART2_TX on pin 62 (MAIN_RXD) — ESP32 IO17 sends commands here

- Add net label UART2_RX on pin 63 (MAIN_TXD) — EC200U sends responses here → ESP32 IO16 receives

- On pin 64 (MAIN_DTR), add net label DTR — connect this to ESP32 IO33 for sleep control

- In ESP32 schematic: verify IO17 = UART2_TX, IO16 = UART2_RX (confirm both match)

# Section 3 — Power Supply Correction

## 3.1 EC200U VBAT Voltage — Critical Fix

The EC200U-CN VBAT (pins 50 = VBAT_BB, 57 = VBAT_RF) operating voltage is 3.4V to 4.3V. Your schematic connects both VBAT pins to VCC_5V (5.0V). This exceeds the absolute maximum rating and WILL permanently damage or destroy the module.

| Pin | Signal | Wrong | Correct | Add Component | KiCad Symbol |
| --- | --- | --- | --- | --- | --- |
| 50 | VBAT_BB | VCC_5V (5.0V) — DESTROYS MODULE | VBAT_4V2 — Output of new 4.2V LDO. Connect through ferrite bead + 100µF cap. | Add: AMS1117-4.2 or HT7342 LDO (SOT-223) | Regulator_Linear:AMS1117-4.2 |
| 57 | VBAT_RF | VCC_5V (5.0V) — DESTROYS MODULE | VBAT_4V2 — Same 4.2V LDO output as VBAT_BB. Both pins share same VBAT rail. | Same 4.2V LDO as above | Same component |
| 128 | VDD_EXT | Connected TO VCC_1V8 — OUTPUT conflict | VDD_EXT is OUTPUT from EC200U (1.8V). Remove VCC_1V8 connection. Use VDD_EXT to power PWRKEY/RESET pull-ups instead. | Remove connection to AMS1117-1.8 output. Use VDD_EXT as supply for pull-ups. | Net label: VDD_EXT_OUT |
| AMS1117-1.8 usage | VCC_1V8 (if kept) | Conflict with VDD_EXT | If AMS1117-1.8 is kept for other purposes, isolate from VDD_EXT pin. Or remove AMS1117-1.8 entirely and use EC200U's VDD_EXT for 1.8V needs. | Optional: Remove AMS1117-1.8 to simplify BOM | Remove if unused |

## 3.2 New Power Circuit — Add AMS1117-4.2 LDO:

| Component | KiCad Symbol | Connection | Notes |
| --- | --- | --- | --- |
| AMS1117-4.2 | Regulator_Linear:AMS1117-4.2 (SOT-223) | VI = VCC_5V, VO = VBAT_4V2, GND = PGND | Provides 4.2V @ 1A. Max EC200U current is ~2A peak. For 2A use LP38501-ADJ. |
| 100µF Cap | Device:C (Electrolytic) | Between VBAT_4V2 and GND — close to AMS1117 output | Handles EC200U 4G TX peak current bursts (up to 2A, ~2ms). |
| 10µF Cap | Device:C (0805 Ceramic) | Between VBAT_4V2 and GND — close to EC200U VBAT pins | Ceramic for fast transient response. |
| 100nF Cap | Device:C (0402 Ceramic) | Between VBAT_4V2 and GND — right at each VBAT pin | High-frequency decoupling. Place within 2mm of pin. |
| Ferrite Bead | Device:Ferrite_Bead (0805) | In series on VBAT_4V2 line before EC200U | 220Ω@100MHz. EMI isolation between power supply and RF module. |

## 3.3 VDD_EXT Pin (128) — Correct Usage:

- VDD_EXT is an OUTPUT from the EC200U module. It provides 1.8V at up to 50mA for external use.

- Disconnect it from VCC_1V8 rail (AMS1117-1.8 output). They will conflict.

- Use VDD_EXT to supply: PWRKEY pull-up resistor (10kΩ from VDD_EXT to PWRKEY), RESET pull-up (10kΩ from VDD_EXT to RESET), USIM card power is provided directly from EC200U internally.

- If you still need 1.8V elsewhere on the board, keep AMS1117-1.8 but isolate it from VDD_EXT completely.

# Section 4 — EC200U Complete Pin Map (All Pins — Verified)

This is the definitive pin-by-pin table based on the exact EC200UCNAA-N05-SGNSA symbol visible in your uploaded schematic. Every pin is documented with its correct connection. Use this as your ground truth when fixing the schematic.

| Pin # | Pin Name | Current Schematic Connection | CORRECT Connection | Net Label | Status |
| --- | --- | --- | --- | --- | --- |
| POWER PINS |  |  |  |  |  |
| 50 | VBAT_BB | VCC_5V → WRONG (5V) | 4.2V LDO output (add AMS1117-4.2 between VCC_5V and this pin) | VBAT_4V2 | ❌ FIX REQUIRED |
| 57 | VBAT_RF | VCC_5V → WRONG (5V) | Same 4.2V LDO rail as VBAT_BB (share same rail) | VBAT_4V2 | ❌ FIX REQUIRED |
| 128 | VDD_EXT | Connected TO VCC_1V8 — CONFLICT | VDD_EXT is an OUTPUT (EC200U provides 1.8V here). Use VDD_EXT to power other 1.8V devices OR leave NC. NEVER connect to LDO output. | VDD_EXT_OUT | ⚠ FIX REQUIRED |
| 8 / GND_1 | GND | → GND | → PGND copper pour | GND | ✔ Correct |
| 85 / GND_2 | GND | → GND | → PGND copper pour | GND | ✔ Correct |
| 101 / GND_3 | GND | → GND | → PGND copper pour | GND | ✔ Correct |
| CONTROL PINS |  |  |  |  |  |
| 21 | PWRKEY | MODULE_PWR ← ESP32 IO26 ✔ | Correct. Add 10kΩ pull-up from PWRKEY to VDD_EXT (1.8V) to keep HIGH when GPIO is floating. | MODULE_PWR | ✔ Correct |
| 20 | RESET | MODULE_RESET ← ESP32 IO27 ✔ | Correct. Add 10kΩ pull-up from RESET to VDD_EXT (1.8V) to keep deasserted (HIGH). | MODULE_RESET | ✔ Correct |
| 19 | STATUS | Net = NMODULE_STATUS (name mismatch vs ESP32 schematic = MODULE_STATUS) | Rename net to MODULE_STATUS for consistency. Connect to ESP32 IO35 input. HIGH = module ON. | MODULE_STATUS | ⚠ Rename Net |
| 3 | SLEEP_IND | NC — floating | Optional: connect to ESP32 GPIO (input). LOW = module in sleep. Leave NC if not monitoring sleep state. | SLEEP_IND | NC — Optional |
| 2 | AP_READY | NC — floating | Optional handshake. Can tie HIGH to VDD_EXT (1.8V) via 47kΩ or leave NC. | AP_READY | NC — OK |
| 190 | ISINK | NC | LED current sink (if module has LED). Leave NC. | — | NC |
| 1 | WAKEUP_IN | NC — floating (dangerous) | REQUIRED: Pull HIGH via 47kΩ to VDD_EXT (1.8V). If left floating, module may not wake properly. | VDD_EXT via 47kΩ | ⚠ Add pull-up |
| 4 | V_DISABLE | NC | Pull HIGH to VDD_EXT (1.8V) via 10kΩ. HIGH = module RF enabled. LOW disables RF. | VDD_EXT via 10kΩ | ⚠ Add pull-up |
| 23 | SIM_DET | NC — not connected to SIM holder | Connect to SIM card holder card-detect pin. 47kΩ pull-up to VDD_EXT. LOW = SIM inserted. | USIM_DET | ⚠ Connect to SIM |
| 5 | NET_MODE | NC | Optional LED indicator for network mode. 10kΩ series to LED → GND. Or leave NC. | — | NC — Optional |
| 6 | NET_STATUS | NC | Optional LED indicator for network registration. Drives LED to show 2G/4G connection status. | — | NC — Optional |
| 439 | BY_EN | NC | Bypass enable — module internal. Leave NC. | — | NC |
| MAIN UART — AT Commands & 4G Data (CRITICAL CORRECTIONS BELOW) |  |  |  |  |  |
| 64 | MAIN_DTR | UART2_TX connected here — WRONG! DTR ≠ UART data | MAIN_DTR = modem control signal (sleep control). Connect to ESP32 IO33 (DTR). If sleep not used, leave NC or pull LOW. | DTR | ❌ WRONG — Move UART2_TX to pin 62 |
| 62 | MAIN_RXD | UART2_RX connected here — WRONG! RXD = EC200U receives | MAIN_RXD = EC200U receives data. Must connect to ESP32 UART2_TX (IO17). NOT to ESP32 RX pin. | UART2_TX (from ESP32 IO17) | ❌ WRONG — Change to UART2_TX |
| 63 | MAIN_TXD | NC — Floating! No connection | MAIN_TXD = EC200U transmits data. MUST connect to ESP32 UART2_RX (IO16). Critical — ESP32 receives all AT responses here. | UART2_RX (to ESP32 IO16) | ❌ FIX — Must be connected! |
| 61 | MAIN_RTS | NC — floating | Tie to GND to disable hardware flow control (most common). OR connect to ESP32 CTS if using flow control. | GND (tie) | ⚠ Tie to GND |
| 65 | MAIN_DCD | NC | Data Carrier Detect — output from module. Not needed for AT command usage. Leave NC. | — | NC — OK |
| 66 | MAIN_RI | NC | Ring Indicator — output, LOW on incoming call/SMS. Connect to ESP32 IO34 (input) for wake-up, or leave NC. | RING_IND → IO34 | Optional |
| GNSS UART — GPS/NMEA Data |  |  |  |  |  |
| 47 | ANT_GNSS | No net/connection shown in schematic! | MUST connect to J3 (SMA_Edge_SMD) via 50Ω controlled impedance microstrip trace. GNSS antenna is mandatory. | GNSS_ANT → J3 | ⚠ Connect to J3 SMA |
| Implied | GNSS_TXD | GNSS_DATA → ESP32 IO13 (schematic shows this) | GNSS_TXD (EC200U transmits NMEA) → ESP32 IO13 (UART1 RX). Correct direction. | GNSS_DATA | ✔ Correct |
| Implied | GNSS_RXD | GNSS_CMD → ESP32 IO12 (schematic shows this) | GNSS_RXD (EC200U GNSS receives config) ← ESP32 IO12 (UART1 TX). Correct direction. | GNSS_CMD | ✔ Correct |
| LTE ANTENNA |  |  |  |  |  |
| 48 | ANT_MAIN | No net/connection shown in schematic! | MUST connect to J2 (SMA_Edge_SMD) via 50Ω controlled impedance microstrip trace. LTE antenna is mandatory. | LTE_ANT → J2 | ⚠ Connect to J2 SMA |
| 35 | ANT_BT/WIFI_SCAN | NC | Not used. Leave NC in this design (no BT/WiFi functionality). | — | NC — OK |
| SIM CARD (USIM) — Connects to J1 SIM Card Holder |  |  |  |  |  |
| 9 | USIM_VDD | → SIM holder VCC pin (J1 pin 1) | EC200U supplies 1.8V or 3V to SIM. Add 47pF decoupling to GND on SIM side. | USIM_VCC | ✔ Connect to SIM |
| 13 | USIM_DET | NC — not connected | Connect to SIM holder card-detect switch. 47kΩ pull-up to VDD_EXT. LOW when SIM inserted. | USIM_DET | ⚠ Connect |
| 15 | USIM_DATA | → SIM holder I/O pin (J1 pin 7) | Bidirectional data. Add 22Ω series resistor + 47pF to GND for ESD protection. | USIM_DATA | ✔ Connect to SIM |
| 16 | USIM_CLK | → SIM holder CLK pin (J1 pin 3) | Clock output to SIM. Add 22Ω series + 47pF to GND. | USIM_CLK | ✔ Connect to SIM |
| 17 | USIM_RST | → SIM holder RST pin (J1 pin 2) | Active LOW reset to SIM. Add 22Ω series. | USIM_RST | ✔ Connect to SIM |
| 18 | USIM_GND | → SIM holder GND pin (J1 pin 5) | Ground for SIM card. Must connect. | GND | ✔ Connect to SIM |
| USB 2.0 (EC200U Internal — Optional) |  |  |  |  |  |
| 71 | USB_VBUS | NC | 5V input if using EC200U USB. NC if using UART-only mode (AT over UART). Leave NC. | — | NC — Using UART |
| 115 | USB_BOOT | NC | Tie to GND for normal operation. HIGH only for EC200U USB firmware update mode. | GND (tie) | ⚠ Tie to GND |
| 69 | USB_DN | NC | USB D-. Leave NC if not using USB port. | — | NC — OK |
| 68 | USB_DP | NC | USB D+. Leave NC if not using USB port. | — | NC — OK |
| I2C Port (EC200U Internal — NC in this design) |  |  |  |  |  |
| 42 | I2C_SDA | NC | EC200U's own I2C port. NOT the ESP32 I2C bus. Leave NC — ESP32 manages all I2C peripherals directly. | — | NC — OK |
| 41 | I2C_SCL | NC | EC200U's own I2C clock. Leave NC. | — | NC — OK |
| 142 | I2C2_SDA | NC | Second I2C port on EC200U. Leave NC. | — | NC — OK |
| 154 | I2C2_SCL | NC | Second I2C clock. Leave NC. | — | NC — OK |
| ADC Pins (Analog Inputs) |  |  |  |  |  |
| 44 | ARCD (ADC0) | NC | Can use for battery voltage monitoring. Connect through 100kΩ/10kΩ divider from VIN_12V. Or NC. | — | NC — Optional |
| 43 | ADC1 | NC | Spare ADC. Leave NC. | — | NC |
| 52 | ADC2 | NC | Spare ADC. Leave NC. | — | NC |
| AUX UART (Secondary AT Port — NC in this design) |  |  |  |  |  |
| 137 | AUX_RXD | NC | Tie HIGH via 47kΩ to VDD_EXT (idle HIGH for UART RX). NC if not using secondary AT port. | VDD_EXT 47kΩ or NC | NC — OK |
| 138 | AUX_TXD | NC | Leave NC if not using secondary UART. Can connect to logic analyzer test point. | — | NC — OK |
| 11 | DBG_RXD | NC | EC200U internal debug UART RX. Leave NC in production. | — | NC |
| 12 | DBG_TXD | NC | EC200U debug output. NC in production (can add test point during development). | — | NC |
| Keypad Interface (NC in this design) |  |  |  |  |  |
| 78 | KEYIN1 | NC | Not used. Leave NC. | — | NC |
| 79 | KEYIN2 | NC | Not used. Leave NC. | — | NC |
| 80 | KEYIN3 | NC | Not used. Leave NC. | — | NC |
| 83 | KEYOUT1 | NC | Not used. Leave NC. | — | NC |
| 84 | KEYOUT2 | NC | Not used. Leave NC. | — | NC |
| 113 | KEYOUT3 | NC | Not used. Leave NC. | — | NC |
| 111 | KEYOUT4 | NC | Not used. Leave NC. | — | NC |
| Audio / Microphone (NC in this design) |  |  |  |  |  |
| 76 | MIC_P | NC | Microphone positive. Not needed for OBD tracker. Leave NC. | — | NC |
| 77 | MIC_N | NC | Microphone negative. Not needed. Leave NC. | — | NC |
| 73 | LOUD5PK_P | NC | Speaker output positive. Not used. Leave NC. | — | NC |
| 74 | LOUD5PK_N | NC | Speaker output negative. Not used. Leave NC. | — | NC |
| PCM Audio Interface (NC in this design) |  |  |  |  |  |
| 27 | PCM_CLK | NC | PCM clock. Not used. Leave NC. | — | NC |
| 24 | PCM_DIN | NC | PCM data in. Not used. Leave NC. | — | NC |
| 25 | PCM_SYNC | NC | PCM sync. Not used. Leave NC. | — | NC |
| 26 | PCM_DOUT | NC | PCM data out. Not used. Leave NC. | — | NC |
| LCD Interface (NC in this design) |  |  |  |  |  |
| 121 | LCD_SEL | NC | LCD select. Not used. | — | NC |
| 119 | LCD_FMARK | NC | LCD frame mark. Not used. | — | NC |
| 122 | LCD_CS | NC | LCD chip select. Not used. | — | NC |
| 120 | LCD_RSTB | NC | LCD reset. Not used. | — | NC |
| 124 | LCD_SDC | NC | LCD serial data clock. Not used. | — | NC |
| 123 | LCD_SI/O | NC | LCD serial I/O. Not used. | — | NC |
| WLAN / WiFi (NC — not applicable for EC200U standard variant) |  |  |  |  |  |
| 136 | VLAN_EN | NC | Pull LOW or leave NC. EC200U standard variant has no WiFi. | GND | Pull LOW |
| 127 | WLAN_PWR_EN | NC | Leave NC. | — | NC |
| 118 | WLAN_SLP_CLK | NC | Leave NC. | — | NC |
| 135 | WLAN_WAKE | NC | Leave NC. | — | NC |
| SPI Flash Port (EC200U Internal — NC) |  |  |  |  |  |
| 50 area | SPL_CLK | NC | SPI flash clock — internal to module. Leave NC. | — | NC |
| ~37 | SPL_CS | NC | SPI flash chip select — internal. Leave NC. | — | NC |
| ~38 | SPL_DOUT | NC | SPI flash data out — internal. Leave NC. | — | NC |
| ~39 | SPL_DIN | NC | SPI flash data in — internal. Leave NC. | — | NC |
| SDIO1 (External SD card — NC in this design) |  |  |  |  |  |
| 34 | SDIO1_VDD | NC | SDIO card power. NC if no SD card. | — | NC |
| 31 | SDIO1_DATA0 | NC | Leave NC. | — | NC |
| 30 | SDIO1_DATA1 | NC | Leave NC. | — | NC |
| 29 | SDIO1_DATA2 | NC | Leave NC. | — | NC |
| 28 | SDIO1_DATA3 | NC | Leave NC. | — | NC |
| 33 | SDIO1_CMD | NC | Leave NC. | — | NC |
| 32 | SDIO1_CLK | NC | Leave NC. | — | NC |
| SDIO2 (Second SDIO — NC in this design) |  |  |  |  |  |
| 132 | SDIO2_DATA0 | NC | Leave NC. | — | NC |
| 134 | SDIO2_DATA1 | NC | Leave NC. | — | NC |
| 130 | SDIO2_DATA2 | NC | Leave NC. | — | NC |
| 129 | SDIO2_DATA3 | NC | Leave NC. | — | NC |
| 131 | SDIO2_CMD | NC | Leave NC. | — | NC |
| 133 | SDIO2_CLK | NC | Leave NC. | — | NC |
| RF Decoupling Pins — MANDATORY |  |  |  |  |  |
| 143 | GRFC1 | No 100nF cap visible in schematic | MANDATORY: Add 100nF capacitor from pin 143 to GND as close as possible to module. | GND via 100nF | ⚠ ADD CAP |
| 139 | GRFC2 | No 100nF cap visible in schematic | MANDATORY: Add 100nF capacitor from pin 139 to GND as close as possible to module. | GND via 100nF | ⚠ ADD CAP |
| General Purpose IO |  |  |  |  |  |
| 126 | GPIO1 | NC | Optional GPIO — can drive an LED (10kΩ series) or leave NC. | — | NC — Optional |
| Reserved Pins — DO NOT CONNECT |  |  |  |  |  |
| 18 area | RESERVED | Must be NC | NEVER connect reserved pins to any net — this can permanently damage the module. | NC — MANDATORY | ❌ Must stay NC |

# Section 5 — Antenna Connection Details

## 5.1 LTE Main Antenna (ANT_MAIN — Pin 48 → J2)

The LTE antenna trace is one of the most critical PCB layout elements. An incorrect trace impedance will significantly reduce signal strength and may cause regulatory non-compliance.

| Item | Specification |
| --- | --- |
| EC200U Pin | Pin 48 — ANT_MAIN |
| Connector | J2 — SMA_Edge_SMD (SMA edge-mount at board edge) |
| Trace Type | 50Ω controlled impedance microstrip on 4-layer board |
| Trace Width | ~1.8mm on standard FR4, 4-layer, 35µm copper, L1-to-L2 reference (calculate with AppCAD or Saturn PCB Toolkit) |
| Trace Length | Keep as short as possible. Route on top layer with unbroken ground plane on L2 below. |
| Ground Stitching | Via fence every 3mm along both sides of RF trace. Keep-out zone 3mm from all other traces. |
| RF Matching (R1+C2) | Optional pi-network between ANT_MAIN and SMA center pin. For dev boards: 0Ω series (R1) + 100nF shunt (C2). For production: tune with VNA. |
| Antenna Type | External LTE rubber duck or PCB antenna, 700MHz–2600MHz, 50Ω |

## 5.2 GNSS Antenna (ANT_GNSS — Pin 47 → J3)

| Item | Specification |
| --- | --- |
| EC200U Pin | Pin 47 — ANT_GNSS |
| Connector | J3 — SMA_Edge_SMD (SMA edge-mount at board edge) |
| Trace Type | 50Ω controlled impedance microstrip |
| Active vs Passive Antenna | Active antenna (built-in LNA): requires DC bias on SMA center pin (3.3V through ferrite bead 10Ω@100MHz + 100nF series capacitor). Passive antenna: no bias needed. |
| Bias-T for Active | Ferrite bead (100nF cap block) on signal path + 3.3V feed through ferrite on center pin. Add to schematic between J3 and ANT_GNSS net. |
| Cold Start Fix Time | ~30s (open sky, GNSS powered). Keep antenna away from metal objects. |

# Section 6 — SIM Card Full Connection

| EC200U Pin | Signal | SIM Holder Pin | Series R | Notes |
| --- | --- | --- | --- | --- |
| 9 | USIM_VDD | Pin C1 (VCC) | 0Ω (direct) | EC200U auto-detects 1.8V or 3V SIM. Add 47pF to GND on SIM side. |
| 17 | USIM_RST | Pin C2 (RST) | 22Ω | Active LOW. 47pF to GND near SIM holder. |
| 16 | USIM_CLK | Pin C3 (CLK) | 22Ω | Clock. 47pF to GND near SIM holder. |
| Pin C4 (reserved) | — | NC | NC | Leave floating — not connected to EC200U. |
| SIM GND | GND | Pin C5 (GND) | 0Ω | Connect to DGND plane. |
| Pin C6 (VPP) | — | NC | NC | Not used for nano/micro SIM — leave NC. |
| 15 | USIM_DATA | Pin C7 (I/O) | 22Ω | Bidirectional. 47pF to GND near SIM. |
| 23 | USIM_DET | Detect switch | — | 47kΩ pull-up to VDD_EXT. LOW = SIM card inserted. |
| SIM shield pins | Metal shell | Holder GND tabs | 0Ω | Connect SIM holder metal shell to DGND — important for ESD/EMC. |

⚠  WARNING:  Add ESD protection on SIM lines. Recommended: PRTR5V0U2X (NXP, SC-74) on USIM_DATA and USIM_CLK. Place within 10mm of SIM holder.

# Section 7 — Module Power-On Sequence (Firmware Reference)

The EC200U-CN requires a specific power-on sequence. If this is not followed in firmware, the module will not respond.

| Step | Action | Signal / GPIO | Timing |
| --- | --- | --- | --- |
| 1 | Apply VBAT_4V2 power | VBAT_BB + VBAT_RF = 4.2V | Power stable before PWRKEY pulse |
| 2 | Confirm STATUS = LOW | ESP32 IO35 reads LOW | Module is off. STATUS=LOW means module not yet running. |
| 3 | Assert PWRKEY LOW | ESP32 IO26 → drive LOW (0V) | Hold LOW for minimum 500ms (recommend 600ms) |
| 4 | Release PWRKEY HIGH | ESP32 IO26 → release HIGH (via 10kΩ pull-up) | Release after 600ms. Module begins boot. |
| 5 | Wait for STATUS=HIGH | ESP32 IO35 reads HIGH | Wait up to 5 seconds. HIGH = module boot complete. |
| 6 | Wait for 'RDY' or 'APP_RDY' | UART2_RX (IO16) from MAIN_TXD (pin 63) | Module sends 'RDY\r\n' when ready for AT commands. |
| 7 | Send AT command | UART2_TX (IO17) → pin 62 (MAIN_RXD) | Send 'AT\r\n'. Should receive 'OK\r\n' back on IO16. |
| 8 | Normal operation | All UART/GNSS active | Begin AT+CREG?, AT+CGNSPWR=1, etc. |

ℹ  Power-off sequence: Assert PWRKEY LOW for minimum 2500ms (2.5 seconds), then release. Wait for STATUS=LOW. Do NOT remove VBAT while module is running — may corrupt internal flash.

# Section 8 — Net Label Consistency Check (ESP32 ↔ EC200U)

| Signal Function | ESP32 GPIO | ESP32 Net Label | EC200U Pin | EC200U Net Label (Correct) |
| --- | --- | --- | --- | --- |
| 4G UART TX | IO17 | UART2_TX | Pin 62 MAIN_RXD | UART2_TX (match exactly) |
| 4G UART RX | IO16 | UART2_RX | Pin 63 MAIN_TXD | UART2_RX (match exactly) |
| Module Power Key | IO26 | MODULE_PWR | Pin 21 PWRKEY | MODULE_PWR (match) |
| Module Reset | IO27 | MODULE_RESET | Pin 20 RESET | MODULE_RESET (match) |
| Module Status | IO35 | MODULE_STATUS | Pin 19 STATUS | MODULE_STATUS (rename from NMODULE_STATUS) |
| DTR Sleep Control | IO33 | DTR | Pin 64 MAIN_DTR | DTR (match) |
| Ring Indicator | IO34 | RING_IND | Pin 66 MAIN_RI | RING_IND (match) |
| GNSS Data (RX) | IO13 | GNSS_DATA | GNSS_TXD pin | GNSS_DATA (match) |
| GNSS CMD (TX) | IO12 | GNSS_CMD | GNSS_RXD pin | GNSS_CMD (match) |
| VBAT | — | VBAT_4V2 (new) | Pin 50+57 | VBAT_4V2 (NEW — not VCC_5V) |
| 1.8V reference | — | VDD_EXT_OUT (new) | Pin 128 | VDD_EXT_OUT (rename — it's OUTPUT) |

✔  Once all net labels are consistent between ESP32 sheet and EC200U sheet, run KiCad ERC (Electrical Rules Check). There should be zero errors on these nets.