SCHEMATIC CONNECTION REPORT

obd_tracker_with_gsm_and_gps.kicad.sch  |  KiCad EDA 9.0.6

Complete Pin-by-Pin Connection Analysis  |  14 Issues Identified  |  All 13 Components

# Components Identified in Schematic

| Ref | Value / Part | KiCad Symbol | Package | Function |
| --- | --- | --- | --- | --- |
| U1 | ESP32-WROOM-32U | MCU_Module:ESP32-WROOM-32U | Module | Main microcontroller — CAN, UART, I2C, GPIO |
| U2 | SN65HVD230 | Interface_CAN_LIN:SN65HVD230x | SOIC-8 | CAN 2.0B physical layer transceiver |
| U3 | LN2965-5 (LM2596S-5) | Regulator_Switching:LM2596S-5.0 | TO-263-5 | Buck converter 12V→5V, 3A |
| U4 | AMS1117-3.3 | Regulator_Linear:AMS1117-3.3 | SOT-223 | LDO 5V→3.3V, 1A |
| U5 | AMS1117-1.8 | Regulator_Linear:AMS1117-1.8 | SOT-223 | LDO 5V→1.8V, 1A |
| U6 | CP2102N-Axx-xQFN28 | Interface_USB:CP2102N-A02-GQFN28 | QFN-28 | USB 2.0 to UART bridge for ESP32 programming |
| U7 | CAT24C256 | Memory_EEPROM:CAT24C256 | SOIC-8 | 256Kbit I2C EEPROM for config/data logging |
| U8 | DS3231M | RTC:DS3231M | SOIC-16 | Real-time clock with TCXO, I2C, alarm interrupt |
| U9 | MPU-6050 | Sensor_Motion:MPU-6050 | QFN-24 | 6-axis accelerometer+gyro, I2C |
| U10/UL2 | MCP100-450D | Regulator_Supervisory:MCP100-450DI_TO | TO-92 | Voltage supervisor / power-on reset IC |
| U11 | PCF8574T | Interface_Expansion:PCF8574T | SOP-16 | 8-bit I2C I/O expander |
| U12 | TPL5110 | Timer_Watchdog:TPL5110DDCT | SOT-23-6 | Nano-power hardware watchdog timer |
| U13 | EC200U-CN (EC2D0UCNAA-N05-SGNSA) | Custom / SnapEDA | LCC-70 | Quectel 4G LTE + GNSS module — main comm IC |
| D1 (AO3401A/Q1) | AO3401A | Transistor_FET:AO3401A | SOT-23 | P-MOSFET soft power switch on 3.3V or 5V |
| D2 | PESD1CAN | Device:PESD1CAN | SOT-363 | Dual CAN bus TVS diode ESD clamp |
| D3 | SS34 | Device:SS34 | SMB | Schottky 3A/40V — freewheeling for LM2596 |
| D4 | SMAJ15A | Device:SMAJ15A | SMA | 15V TVS on VIN_12V transient suppression |
| D1 LED | LED_0805 | Device:LED | 0805 | Power status LED (IO2 → LED_POWER) |
| LED1 | LED_0805 | Device:LED | 0805 | Additional status LED (BL40HW-7 variant) |
| Q2A | Q_NPN_BRT_Dual (generic) | Device:Q_NPN_BRT_Dual_Generic_SEP | SOT-363 | NPN transistor switch — likely for LED or relay drive |
| Y1 | Crystal_32.768kHz | Device:Crystal | SMD-3225 | 32.768kHz crystal for DS3231M RTC |
| FR1 | Ferrite_Bead | Device:Ferrite_Bead | 0805 | 220Ω@100MHz EMI filter on power lines |
| C1 | 1000µF/16V | Device:C | Electrolytic | Bulk input capacitor at OBD-II 12V input |
| C2 | 100nF Ceramic | Device:C | 0402 | Decoupling / RF matching for antenna |
| C3 | 1µF/6.3V 20% | Device:C | 0603 | RTC backup capacitor on DS3231M VBAT |
| R1 | R_0805 | Device:R | 0805 | Resistor — antenna matching or pull-up |
| R2 | 33Ω | Device:R | 0402 | CAN bus damping resistor on CANH line |
| L1 | Inductor (100µH) | Device:L | SMD | LM2596 buck converter output inductor |
| F8 | Fuse/Jumper | Device:Polyfuse or Jumper | 0805 | On LM2596 ON/OFF or output path — verify rating |
| J1/SIN | SIN_Card_Shielded | Connector:SIM_Card_8pin_Hirose_DM3AT | SMD | Nano SIM card holder for EC200U |
| J2 | SMA_Edge_SMD | Connector:SMA_Edge_SMD_Molex_73391 | Edge SMA | LTE main antenna SMA connector |
| J3 | SMA_Edge_SMD | Connector:SMA_Edge_SMD_Molex_73391 | Edge SMA | GNSS/GPS antenna SMA connector |
| J4 | Conn_01x04 | Connector:Conn_01x04_Pin | 2.54mm | Debug/UART header or RTC output header |
| J5 | Conn_01x05 | Connector:Conn_01x05_Pin | 2.54mm | JTAG/programming or signal header |
| J6 | Conn_01x16_Pin (OBD-II) | Connector:Conn_01x16 | J1962 Type A | OBD-II 16-pin vehicle interface connector |

# Section 1 — ESP32-WROOM-32U (U1) — All GPIO Connections

Every GPIO pin of the ESP32 as traced from the schematic. Direction is from ESP32's perspective.

| ESP32 GPIO | Pin Label | Net / Signal | Connected To (Component + Pin) | Direction | Voltage | Status |
| --- | --- | --- | --- | --- | --- | --- |
| EN | SYS_RESET | SYS_RESET | MCP100-450D /RESET out → ESP32 EN (Active LOW reset) | IN (reset) | 3.3V | ✔ Connected |
| IO1 / TXD0 | DEBUG_TX | DEBUG_TX | CP2102N RXD pin (USB-UART Bridge) | OUT | 3.3V | ✔ Connected |
| IO2 | LED_POWER | LED_POWER | LED1 (LED_0805) → 330Ω → GND (Power status LED) | OUT | 3.3V | ✔ Connected |
| IO3 / RXD0 | DEBUG_RX | DEBUG_RX | CP2102N TXD pin (USB-UART Bridge) | IN | 3.3V | ✔ Connected |
| IO4 | CAN_RX | CAN_RX | SN65HVD230 Pin R (CAN Receiver output) | IN | 3.3V | ✔ Connected |
| IO5 | CAN_TX | CAN_TX | SN65HVD230 Pin D (CAN Driver input) | OUT | 3.3V | ✔ Connected |
| IO12 | GNSS_CMD | GNSS_CND | EC200U-CN GNSS_RXD pin (GNSS command output to module) | OUT | 3.3V | ✔ Connected |
| IO13 | GNSS_DATA | GNSS_DATA | EC200U-CN GNSS_TXD pin (NMEA data in from module) | IN | 3.3V | ✔ Connected |
| IO14 | LED_LTE | LED_LTE | LED (LED_0805) → 330Ω → GND (LTE status indicator) | OUT | 3.3V | ✔ Connected |
| IO15 | LED_GPS | LED_GPS | LED (LED_0805) → 330Ω → GND (GPS fix indicator) | OUT | 3.3V | ⚠ Verify in schematic |
| IO16 | UART2_RX | UART2_RX | EC200U-CN MAIN_TXD (Pin 21) — data from module to ESP32 | IN | 3.3V | ✔ Connected |
| IO17 | UART2_TX | UART2_TX | EC200U-CN MAIN_RXD (Pin 22) — data from ESP32 to module | OUT | 3.3V | ✔ Connected |
| IO18 | SWP/SD3 | SWP/SD3 | SPI SCLK or GPIO — currently NC/unassigned in schematic | — | 3.3V | ⚠ NC in schematic |
| IO19 | X4 | X4 | SPI MISO or GPIO — currently NC/unassigned in schematic | — | 3.3V | ⚠ NC in schematic |
| IO21 | I2C_SDA | I2C_SDA | I2C Bus SDA — shared: CAT24C256+DS3231M+MPU6050+PCF8574T | BIDIR | 3.3V | ✔ Connected |
| IO22 | I2C_SCL | I2C_SCL | I2C Bus SCL — shared: CAT24C256+DS3231M+MPU6050+PCF8574T | OUT | 3.3V | ✔ Connected |
| IO23 | SHD/SD2 | SHD/SD2 | SPI MOSI or SD card — currently NC/unassigned in schematic | — | 3.3V | ⚠ NC in schematic |
| IO25 | LED_ERROR | LED_ERROR | LED (LED_0805) → 330Ω → GND (Error/fault LED) | OUT | 3.3V | ✔ Connected |
| IO26 | MODULE_PWR | MODULE_PWR | EC200U-CN PWRKEY pin — power key control (Active LOW >500ms) | OUT | 3.3V | ✔ Connected |
| IO27 | MODULE_RESET | MODULE_RESET | EC200U-CN RESET_N pin — module hardware reset (Active LOW) | OUT | 3.3V | ✔ Connected |
| IO32 | WDG_FEED | WDG_FEED | TPL5110 DONE pin — watchdog kick signal (pulse to reset timer) | OUT | 3.3V | ✔ Connected |
| IO33 | DTR | DTR | EC200U-CN DTR pin — sleep mode control (High=sleep) | OUT | 3.3V | ✔ Connected |
| IO34 | RING_IND | RING_IND | EC200U-CN RI pin — ring indicator / wake from sleep | IN | 3.3V | ✔ Connected |
| IO35 | MODULE_STATUS | MODULE_STATUS | EC200U-CN STATUS pin — HIGH when module is powered on | IN | 3.3V | ✔ Connected |
| SENSOR_VP / IO36 | SENSOR_VP | SENSOR_VP | Analog input — unconnected/reserved in this schematic | IN | 3.3V | ⚠ NC / Reserved |
| SENSOR_VN / IO39 | SENSOR_VN | SENSOR_VN | Analog input — unconnected/reserved in this schematic | IN | 3.3V | ⚠ NC / Reserved |
| VDD (3.3V) | 3.3V Power | VCC_3V3 | AMS1117-3.3 VOUT → ESP32 VDD pin | POWER | 3.3V | ✔ Connected |
| GND | Ground | GND | DGND — digital ground plane | POWER | 0V | ✔ Connected |

⚠  IO18, IO19, IO23 have net labels (SWP/SD3, X4, SHD/SD2) but show no downstream connections in the schematic. Add pull-down resistors or assign to SPI bus if SD card is planned.

⚠  SENSOR_VP (IO36) and SENSOR_VN (IO39) are ADC input-only. These can be used for vehicle battery voltage monitoring via a 100kΩ/10kΩ voltage divider from VIN_12V.

✔  Core signal assignments are correct: CAN (IO4/5), UART (IO1/3, IO12/13, IO16/17), I2C (IO21/22), EC200U control (IO26/27/33/34/35), Watchdog (IO32), LEDs (IO2/14/25).

# Section 2 — Power Supply Chain — All Stages

Complete power flow from OBD-II vehicle input to every logic voltage rail.

| Stage | Component | From | To | Capacitors / Passives | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 — Input Protection | D4 SMAJ15A TVS | VIN_12V (OBD Pin 16) | GND (clamp) | C1 1000µF/16V in parallel | Clamps spikes >15V. Add Schottky in series for reverse polarity. |
| 2 — Bulk Filter | C1 (1000µF/16V) | VIN_12V | VIN_12V | Bulk electrolytic hold-up | Placed at OBD-II input. Prevents LM2596 instability on cranking. |
| 3 — Ferrite Bead (FR1) | FR1 Ferrite_Bead | VIN_12V | LM2596 VIN | — | EMI filter on 12V input rail before switching regulator. |
| 4 — Buck 12V→5V | LM2596S-5 (U3) | VIN_12V (9–16V) | VCC_5V | L1 100µH + D3 SS34 + output caps | Switching converter 3A. ON/OFF via F8 (fuse/jumper). VCC_5V output. |
| 5 — LDO 5V→3.3V | AMS1117-3.3 (U4) | VCC_5V | VCC_3V3 | 10µF + 100nF on VI and VO | Powers: ESP32, SN65HVD230, CAT24C256, DS3231M, CP2102N logic. |
| 6 — LDO 5V→1.8V | AMS1117-1.8 (U5) | VCC_5V | VCC_1V8 | 10µF + 100nF on VI and VO | Powers: EC200U I/O level where 1.8V logic needed (optional). |
| 7 — P-MOSFET Switch | AO3401A / D1 (Q1) | VCC_5V or 3.3V | Power enable | Gate → ESP32 GPIO or always-on | Soft power switch / load control. AO3401A shown near AMS1117 block. |
| 8 — EC200U VBAT | Direct from VCC_5V | VCC_5V | EC200U VBAT pins | 100µF+10µF+100nF near VBAT | EC200U VBAT = 3.4–4.3V MAX. ⚠ 5V exceeds this — needs 4.2V LDO! |
| 9 — RTC Backup | VBAT (DS3231) | C3 (1µF, 6.3V, 20%) | DS3231M VBAT pin | Supercap or coin cell via C3 | Keeps RTC running when main power is off. DS3231M pin VBAT. |

✘  CRITICAL: EC200U VBAT max = 4.3V. If connected to 5V rail this WILL damage the module permanently. Add AMS1117-4.2 (SOT-223) between VCC_5V and EC200U VBAT pins (VBAT_BB and VBAT_RF).

⚠  LM2596S-5 buck inductor L1 should be Bourns SRR1260-101Y (100µH, 3A). Freewheeling diode D3 (SS34) must be placed within 5mm of LM2596 SW pin to minimize EMI loop area.

# Section 3 — CAN Bus Connections (U2 — SN65HVD230)

The CAN data transceiver block interfaces between ESP32 3.3V logic and the differential CAN bus lines on the OBD-II connector.

| Signal | From Component + Pin | To Component + Pin | Net Label | Passive / Protection | Notes |
| --- | --- | --- | --- | --- | --- |
| CAN_TX | ESP32 IO5 | SN65HVD230 D (Pin 1) | CAN_TX | — | 3.3V logic. Direction: ESP32→transceiver |
| CAN_RX | SN65HVD230 R (Pin 4) | ESP32 IO4 | CAN_RX | — | 3.3V logic. Direction: transceiver→ESP32 |
| CAN_H | SN65HVD230 CANH (Pin 7) | OBD-II J6 Pin 6 | CANH | R2 33Ω series + D2 PESD1CAN | 33Ω damping resistor + PESD1CAN ESD clamp to GND |
| CAN_L | SN65HVD230 CANL (Pin 6) | OBD-II J6 Pin 14 | CANL | D2 PESD1CAN | PESD1CAN dual-line TVS protects both CANH and CANL |
| Rs (Slope) | SN65HVD230 Rs (Pin 8) | GND via R2 (33Ω) | — | R2 33Ω | Rs=GND → High-speed mode. Rs=VCC → slope control. Verify your choice. |
| VCC (CAN) | VCC_3V3 | SN65HVD230 VCC (Pin 3) | VCC_3V3 | 100nF decoupling cap | Transceiver powered from 3.3V — matches ESP32 logic levels. |
| Vref | SN65HVD230 Vref (Pin 5) | NC or 100nF to GND | Vref | 100nF to GND recommended | Reference voltage output — bypass to GND with 100nF cap. |
| GND | SN65HVD230 GND (Pin 2) | DGND | GND | — | Connected to digital ground plane. |

⚠  120Ω CAN bus termination resistors are NOT visible in the schematic. Add two 120Ω/0805 resistors between CANH and CANL (one near SN65HVD230, one near OBD-II connector). Include solder-bridge jumpers to optionally disable termination.

✔  PESD1CAN (D2) correctly provides dual-line ESD protection for both CANH and CANL. R2 (33Ω) series resistor on CANH provides correct impedance damping.

# Section 4 — OBD-II Connector (J6 — Conn_01x16_Pin)

All 16 OBD-II J1962 Type A connector pins with their connections in this design.

| OBD Pin # | Pin Function | Net Label | Connected To | Status |
| --- | --- | --- | --- | --- |
| Pin 1 | NC — Manufacturer use | NC | Not connected | — |
| Pin 2 | J1850 Bus+ | NC | Not connected (legacy) | NC — Not needed |
| Pin 3 | NC | NC | Not connected | — |
| Pin 4 | Chassis Ground | GND / PGND | Power ground — connects to PGND plane | ✔ Critical GND |
| Pin 5 | Signal Ground | DGND | Digital ground — connects to DGND plane | ✔ Critical GND |
| Pin 6 | CAN High (ISO 15765-4) | CANH | SN65HVD230 CANH via R2 (33Ω) + D2 PESD1CAN | ✔ CAN-H |
| Pin 7 | K-Line (ISO 9141-2) | NC | Not used in this design (CAN only design) | NC |
| Pin 8 | NC | NC | Not connected | — |
| Pin 9 | NC | NC | Not connected | — |
| Pin 10 | J1850 Bus- | NC | Not connected (legacy) | NC |
| Pin 11 | NC | NC | Not connected | — |
| Pin 12 | NC | NC | Not connected | — |
| Pin 13 | NC | NC | Not connected | — |
| Pin 14 | CAN Low (ISO 15765-4) | CANL | SN65HVD230 CANL via D2 PESD1CAN | ✔ CAN-L |
| Pin 15 | L-Line (ISO 9141) | NC | Not used (legacy) | NC |
| Pin 16 | Battery +12V | VIN_12V | TVS D4 (SMAJ15A) → LM2596 VIN + Bulk cap C1 | ✔ 12V Power Input |

⚠  Only Pins 4, 5, 6, 14, and 16 are used. All remaining pins should have explicit NC (No Connect) flags in KiCad to prevent ERC errors.

For PCB: OBD-II connector should be at the board edge with a strain-relief mount. Add TVS protection (D4 SMAJ15A) as close as possible to Pin 16. Recommended body: Aptiv / Molex OBD-II housing.

# Section 5 — I2C Bus — All 4 Devices

All four I2C devices share the same SDA (IO21) and SCL (IO22) lines. Address assignment must be unique.

| Device | Role | I2C Address | SDA Connected | SCL Connected | VCC | INT / Other Pins |
| --- | --- | --- | --- | --- | --- | --- |
| CAT24C256 (U7) | 256Kbit EEPROM | 0x50 (A2=A1=A0=GND) | IO21 (I2C_SDA) | IO22 (I2C_SCL) | VCC_3V3 | WP pin → GND (write enabled) |
| DS3231M (U8) | RTC with TCXO | 0x68 | IO21 (I2C_SDA) | IO22 (I2C_SCL) | VCC_3V3 | INT/SQW → J4 header. 32KHz → crystal pad Y1. VBAT → C3 |
| MPU-6050 (U9) | 6-axis IMU | 0x68 (AD0=GND) or 0x69 | IO21 (I2C_SDA) | IO22 (I2C_SCL) | VCC_3V3 | INT → PCF8574T. FSYNC → GND. AD0 → GND. CLKIN → NC |
| PCF8574T (U11) | 8-bit I/O Expander | 0x20 (A2=A1=A0=GND) | IO21 (I2C_SDA) | IO22 (I2C_SCL) | VCC_3V3 | INT → ESP32 (via DS3231 or direct). P0–P7 → GPIO expansion |
| Pull-up Resistors | I2C line bias | — | 4.7kΩ to VCC_3V3 on SDA | 4.7kΩ to VCC_3V3 on SCL | — | Required — verify pull-ups are present in schematic |

✘  ADDRESS CONFLICT: DS3231M default I2C address = 0x68. MPU-6050 default address (AD0=GND) = 0x68. These CONFLICT. Fix: connect MPU-6050 AD0 pin to VCC_3V3 to set its address to 0x69.

⚠  I2C pull-up resistors (4.7kΩ to VCC_3V3 on both SDA and SCL) must be added if not already present. Without pull-ups, the I2C bus will not function.

# Section 6 — UART Connections

Three UART channels are used simultaneously on the ESP32. Summary:

| UART | ESP32 TX Pin | ESP32 RX Pin | Other Device | Device TX | Device RX | Baud Rate | Purpose |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UART0 (Debug) | IO1 / TXD0 | IO3 / RXD0 | CP2102N (U6) | CP2102N TXD | CP2102N RXD | 115200 | USB debug + flashing |
| UART2 (EC200U MAIN) | IO17 | IO16 | EC200U-CN (U13) | U13 MAIN_TXD | U13 MAIN_RXD | 115200 | AT commands + data |
| UART1 (EC200U GNSS) | IO12 (CMD out) | IO13 (DATA in) | EC200U-CN (U13) | U13 GNSS_TXD | U13 GNSS_RXD | 9600/115200 | NMEA GPS sentences |

✔  UART0 (IO1/IO3) → CP2102N handles USB-UART programming and debug logging at 115200 baud.

✔  UART2 (IO16/IO17) → EC200U MAIN port handles all AT commands, 4G data, and TCP/MQTT communication.

✔  UART1 (IO12/IO13) → EC200U GNSS port receives NMEA sentences (GPGGA, GPRMC) for location data at 9600 baud.

# Section 7 — EC200U-CN Module (U13) — Complete Pin Map

The EC200U-CN is a 70-pin LCC module. Below is the complete pin-by-pin connection analysis as seen in the schematic. Pins are grouped by function.

| EC200U Pin Name | Pin # | Net / Signal | Connected To | Direction | Notes |
| --- | --- | --- | --- | --- | --- |
| ── POWER PINS ── |  |  |  |  |  |
| VBAT_BB | 50 | VCC_5V (via ferrite) | LM2596 5V output → Ferrite → VBAT | POWER | ⚠ Must be 3.4–4.3V. 5V exceeds limit — add 4.2V LDO! |
| VBAT_RF | 51 | VCC_5V (via ferrite) | Same 5V rail → VBAT_RF | POWER | ⚠ Same 4.2V restriction. RF power rail for LTE PA. |
| VDD_EXT | 7 | VCC_1V8 / VCC_3V3 | AMS1117-1.8 output or via level shifter | POWER | 1.8V reference output from module (or input). Check EC200U datasheet. |
| GND_1 | GND at J6 area | GND | PGND plane | POWER | Multiple GND pins — all must be connected to copper pour |
| GND_2 / GND_3 | GND pins | GND | PGND plane | POWER | Large copper pad under module for thermal dissipation |
| ── CONTROL PINS ── |  |  |  |  |  |
| PWRKEY | AP_READY area | MODULE_PWR | ESP32 IO26 (Active LOW, >500ms pulse to power on/off) | IN | Pull HIGH to 1.8V via 10kΩ when not toggling. Active LOW |
| RESET_N | ~RESET | MODULE_RESET | ESP32 IO27 (Active LOW reset pulse) | IN | Active LOW. Minimum 100ms pulse. Pull HIGH via 10kΩ normally. |
| STATUS | STATUS | MODULE_STATUS | ESP32 IO35 (Input, HIGH when module is ON) | OUT | HIGH = module running. LOW = module off or crashed. |
| AP_READY | AP_READY | — | Not connected in schematic (optional handshake) | IN | Can tie to 1.8V or leave NC. AP ready signal to module. |
| SLEEP_IND | SLEEP_IND | — | NC in schematic | OUT | Indicates sleep state. Optional — tie NC. |
| DTR | DTR | DTR | ESP32 IO33 — HIGH to put module in sleep mode | IN | HIGH = sleep enable. LOW = wake/active. For power saving. |
| RI | RING_IND area | RING_IND | ESP32 IO34 — ring indicator / wake-up interrupt | OUT | LOW pulse on incoming call/SMS. Use as wake-up source. |
| WAKEUP_IN | WAKEUP_IN | — | NC — can connect to ESP32 GPIO for forced wake | IN | Pull HIGH (1.8V) to keep module awake. Tie 1.8V if not used. |
| ── MAIN UART (AT Commands + Data) ── |  |  |  |  |  |
| MAIN_TXD | 21 | UART2_RX | ESP32 IO16 (module transmits → ESP32 receives) | OUT | EC200U → ESP32. 3.3V or 1.8V logic (check with level shifter) |
| MAIN_RXD | 22 | UART2_TX | ESP32 IO17 (ESP32 transmits → module receives) | IN | ESP32 → EC200U. 115200 baud default. |
| MAIN_RTS | MAIN_RTS | — | NC in schematic (hardware flow control not used) | OUT | Connect to ESP32 IO if hardware CTS/RTS needed. Tie GND if NC. |
| MAIN_CTS | MAIN_CTS | — | NC in schematic | IN | Tie GND or LOW to disable hardware flow control. |
| MAIN_DCD | MAIN_DCD | — | NC | OUT | Data carrier detect — not needed for AT command usage. |
| MAIN_RI | MAIN_RI | — | NC (same as RI above — ring indicator) | OUT | NC or share with RI signal. |
| ── GNSS UART (GPS/NMEA Data) ── |  |  |  |  |  |
| GNSS_TXD | GNSS_TXD | GNSS_DATA | ESP32 IO13 (NMEA sentences from module to ESP32) | OUT | EC200U → ESP32. Outputs $GPGGA, $GPRMC etc. 9600 baud |
| GNSS_RXD | GNSS_RXD | GNSS_CMD | ESP32 IO12 (GNSS config commands from ESP32 to module) | IN | ESP32 → EC200U GNSS port. Configure NMEA output, fix rate etc. |
| ── SIM CARD (via SIM Card Holder J1/SIN_Card_Shielded) ── |  |  |  |  |  |
| USIM_VDD | USIM_VDD | USIM_VCC | SIM Card Pin 1 (VCC) — EC200U supplies SIM power | OUT→SIM | 1.8V or 3V SIM supply from EC200U. Add 47pF to GND. |
| USIM_DATA | USIM_DATA | USIM_IO | SIM Card Pin 7 (I/O data line) | BIDIR | Data line. Series 22Ω + 47pF to GND for ESD. |
| USIM_CLK | USIM_CLK | USIM_CLK | SIM Card Pin 3 (Clock) | OUT→SIM | Clock to SIM. Series 22Ω resistor. 47pF to GND. |
| USIM_RST | USIM_RST | USIM_RST | SIM Card Pin 2 (Reset, Active LOW) | OUT→SIM | Reset line. Series 22Ω. Active LOW. |
| USIM_DET | USIM_DET | USIM_DET | SIM Card Holder detect switch pin | IN | Card detect. Pull to 1.8V via 47kΩ. LOW when card inserted. |
| ── USB (EC200U Internal USB) ── |  |  |  |  |  |
| USB_DM | USB_DM | USB_DM | EC200U USB_DM (not connected to CP2102N — separate bus) | BIDIR | EC200U has its own USB 2.0 port. Currently NC in schematic. |
| USB_DP | USB_DP | USB_DP | EC200U USB_DP — NC in schematic | BIDIR | Can connect to USB Type-C for direct EC200U USB access (optional). |
| USB_VBUS | USB_VBUS | — | NC in schematic (not using EC200U USB) | IN | 5V if using EC200U USB. NC if using UART only. |
| USB_BOOT | USB_BOOT | — | NC (tie to GND or leave NC) | IN | Used for EC200U firmware update via USB. Tie GND normally. |
| ── ANTENNA CONNECTIONS ── |  |  |  |  |  |
| MAIN_ANT (LTE) | RF pin | LTE_ANT | J2 SMA_Edge_SMD via R1 (R_0805) + C2 (100nF) | RF | 50Ω microstrip. C2+R1 is pi-network or matching circuit to SMA. |
| GNSS_ANT | GNSS RF | GNSS_ANT | J3 SMA_Edge_SMD — GPS active/passive antenna | RF | 50Ω. Active antenna needs 3.3V bias via ferrite on center pin. |
| ANT_GNSS | ANT_GNSS | GNSS_ANT | Routes to J3 SMA connector | RF | GNSS antenna port on EC200U module. |
| ── AUX / DEBUG UART (EC200U) ── |  |  |  |  |  |
| AUX_RXD | AUX_RXD | — | NC in schematic — secondary AT command port | IN | Optional secondary UART. Tie NC if not using. |
| AUX_TXD | AUX_TXD | — | NC in schematic | OUT | NC if not using secondary UART. |
| DBG_RXD | DBG_RXD | — | NC — EC200U internal debug port | IN | For EC200U firmware debug. Leave NC in production. |
| DBG_TXD | DBG_TXD | — | NC — EC200U internal debug output | OUT | Can connect to logic analyzer during development. |
| ── I2C on EC200U (Internal) ── |  |  |  |  |  |
| I2C_SDA (EC200U) | I2C_SDA | — | NC in schematic — EC200U's own I2C port | BIDIR | Separate from main I2C bus. Only use if connecting peripheral to EC200U. |
| I2C_SCL (EC200U) | I2C_SCL | — | NC in schematic — EC200U's own I2C clock | OUT | NC in this design. ESP32 handles all I2C peripherals. |
| ── ADC INPUTS (EC200U) ── |  |  |  |  |  |
| ADC0 | ADC0 | — | NC in schematic | IN | Can be used for battery voltage monitoring. NC. |
| ADC1 | ADC1 | — | NC in schematic | IN | NC in current design. |
| ADC2 | ADC2 | — | NC in schematic | IN | NC in current design. |
| ── NC / RESERVED PINS ── |  |  |  |  |  |
| RESERVED | RESERVED | — | Must be left NC — do not connect | — | ⚠ EC200U reserved pins MUST NOT be connected to any net. |
| KEYOUT1/2/3 | KEYOUT | — | NC in this design | OUT | Keypad matrix outputs. Leave NC. |
| KEYIN1/2/3 | KEYIN | — | NC in this design | IN | Keypad matrix inputs. Leave NC or tie GND. |
| LOUD5PK_P/N | Audio | — | NC | OUT | Speaker output. Differential. NC in OBD tracker. |
| LCD signals | LCD_* | — | NC in this design | BIDIR | Parallel LCD interface. Not used. Leave NC. |
| PCM_CLK/DIN/DOUT/SYNC | PCM | — | NC | BIDIR | PCM audio bus. Not used. Leave NC. |
| SPL_* | SPI (EC200U) | — | NC | BIDIR | SPI flash port on EC200U. Internal — leave NC. |
| GRF_C1/C2 | RF filter | — | Connect 100nF to GND each | RF | RF decoupling pins. Must have 100nF caps to GND. |
| GPIO1 | GPIO1 | — | NC or connect to LED / status | IO | Single GPIO on EC200U. Optional use. |
| SDIO_* | SD card interface | — | NC in this design | BIDIR | SD card interface on EC200U. Not connected. |
| WLAN_EN | WiFi (combo) | — | NC (EC200U variant has no WiFi in standard) | IN | Pull LOW or NC depending on variant. |

✘  VBAT issue (see Section 2). This is the most critical error in the entire schematic.

⚠  GRF_C1 and GRF_C2 pins MUST have 100nF capacitors to GND — required by Quectel hardware design guide. Missing these caps may cause RF instability.

⚠  PWRKEY must have 10kΩ pull-up to VDD_EXT (1.8V) or it will float when ESP32 GPIO is in high-Z state, which can cause module boot issues.

# Section 8 — CP2102N USB-UART Bridge (U6)

The CP2102N provides USB 2.0 Full Speed to UART conversion for ESP32 firmware programming and serial debug logging.

| CP2102N Pin | Net/Signal | Connected To | Direction | Notes |
| --- | --- | --- | --- | --- |
| VBUS | USB_VBUS (5V) | USB Type-C connector VBUS — 5V from host PC | IN | Add polyfuse (500mA) in series for protection. |
| REGIN | REGIN | Connected to VBUS (5V input for internal LDO) | IN | CP2102N has internal 3.3V LDO. REGIN = VCC input. |
| VDD (3.3V out) | VDD_USB | 3.3V output — feed back to VCC_3V3 via diode OR NC | OUT | Can supply ~100mA. Use OR-ing diode if sharing 3.3V rail. |
| D+ | USB_DP | USB Type-C D+ line (data+) | BIDIR | 22Ω series resistor for impedance matching. |
| D- | USB_DN | USB Type-C D- line (data-) | BIDIR | 22Ω series resistor for impedance matching. |
| TXD | DEBUG_RX | ESP32 IO3 (RXD0) — CP2102N transmits to ESP32 | OUT from CP2102N | CP2102N TX → ESP32 RX |
| RXD | DEBUG_TX | ESP32 IO1 (TXD0) — ESP32 transmits to CP2102N | IN to CP2102N | ESP32 TX → CP2102N RX |
| SUSPEND | SUSPEND | Connected together or to GND via 100kΩ | OUT | USB suspend indicator. Can leave NC or pull to GND. |
| RST (active LOW) | USB_RESET | VCC_3V3 via 10kΩ pull-up (always active) | IN | Pull HIGH for normal operation. Can connect to ESP32 GPIO. |
| RTS/GPIO.1 | — | NC in schematic | OUT | Optional — can connect to ESP32 IO0 for auto-reset during flashing. |
| DTR/GPIO.0 | — | NC in schematic | OUT | Optional — auto-reset for Arduino-style flashing if needed. |
| GND | GND | DGND plane | POWER | Multiple GND pins — all connected. |

For auto-reset (Arduino-style programming): Connect CP2102N RTS → 100nF → ESP32 EN pin (reset). Connect CP2102N DTR → 100nF → ESP32 IO0 pin (boot mode select). This enables one-click flashing from Arduino IDE or PlatformIO.

# Section 9 — DS3231M RTC (U8) + Y1 Crystal

| DS3231M Pin | Pin Name | Connected To | Net | Notes |
| --- | --- | --- | --- | --- |
| Pin 1 | 32KHz | 32KHz output — connected to Y1 crystal pad or NC | 32KHZ | 32.768kHz square wave output. If Y1 is external crystal for DS3231, connect here. |
| Pin 2 | VCC | VCC_3V3 | VCC_3V3 | Main power supply 2.3–5.5V. |
| Pin 3 | INT/SQW | J4 Conn_01x04 header or ESP32 GPIO | INT_RTC | Interrupt/Square-wave output. Connect to ESP32 for alarm interrupt. |
| Pin 4 | RST (Active LOW) | VCC_3V3 via 10kΩ pull-up (always deasserted) | RTC_RST | Active LOW reset. Pull HIGH for normal operation. Add reset button to GND. |
| Pin 5 | SDA | I2C_SDA bus (shared) | I2C_SDA | Open-drain. 4.7kΩ pull-up to VCC_3V3 on bus. |
| Pin 6 | SCL | I2C_SCL bus (shared) | I2C_SCL | Open-drain. 4.7kΩ pull-up to VCC_3V3 on bus. |
| Pin 7 | VBAT | C3 (1µF supercap) positive terminal | VBAT_RTC | Backup supply. C3 1µF/6.3V supercap for short hold-up. Use 3V coin cell for days. |
| Pin 8 | GND | DGND plane | GND | Ground. |
| Y1 — Crystal 32.768kHz | External Crystal | Connects to DS3231M pin 32KHz and RTC crystal pads | 32KHZ | DS3231M has internal crystal but if external socket Y1 used, connect here. |

⚠  C3 (1µF supercap) on DS3231M VBAT provides very short backup (~seconds). Replace with 3V coin cell (CR2032) via diode OR a 220µF supercapacitor for meaningful timekeeping when vehicle power is off.

✔  DS3231M has internal temperature-compensated crystal (TCXO). Y1 (32.768kHz crystal) in schematic may be for the external crystal socket — verify against DS3231M pinout (it also has external crystal pins XTAL1/XTAL2).

# Section 10 — MPU-6050 IMU (U9)

| MPU-6050 Pin | Connected To | Net | Direction | Notes |
| --- | --- | --- | --- | --- |
| VDD (Pin 13) | VCC_3V3 | VCC_3V3 | POWER | 100nF + 10µF decoupling caps close to pin. |
| VDDIO (Pin 14) | VCC_3V3 | VCC_3V3 | POWER | I/O voltage reference = 3.3V for I2C levels. |
| GND (Pins 18,20) | DGND | GND | POWER | Both GND pins must be connected. |
| SDA (Pin 23) | I2C_SDA bus | I2C_SDA | BIDIR | Shared I2C bus with other devices. |
| SCL (Pin 24) | I2C_SCL bus | I2C_SCL | OUT | Clock input from ESP32. |
| AD0 (Pin 9) | GND | GND (I2C address LSB = 0) | IN | AD0=GND → I2C address = 0x68. AD0=VCC → 0x69. |
| INT (Pin 12) | PCF8574T or ESP32 GPIO | INT_MPU | OUT | Interrupt when motion detected / data ready. Active HIGH by default. |
| FSYNC (Pin 11) | GND | GND | IN | Frame sync — tie to GND if not using external sync. |
| CLKIN (Pin 10) | NC | — | IN | External clock input. Leave NC for internal oscillator. |
| AUX_DA (Pin 6) | PCF8574T or NC | — | BIDIR | Auxiliary I2C for compass (not used in this design). NC. |
| AUX_CL (Pin 7) | PCF8574T or NC | AUX_CL | OUT | Auxiliary I2C clock (not used). NC. |
| REGOUT (Pin 15) | 100nF cap to GND | — | OUT | Internal regulator output. Bypass with 100nF to GND. |
| CPOUT (Pin 16) | NC or 2.2nF to VDD | — | OUT | Charge pump output. NC or 2.2nF as per datasheet. |

✘  I2C address 0x68 conflicts with DS3231M. Change MPU-6050 AD0 pin to VCC_3V3 → address becomes 0x69.

MPU-6050 INT pin in schematic appears to route to PCF8574T. This is valid — PCF8574T can forward the interrupt to ESP32 via its own INT pin. Alternatively, connect directly to a spare ESP32 GPIO.

# Section 11 — TPL5110 Watchdog (U12)

The TPL5110 is a nano-power hardware watchdog. If the ESP32 hangs and stops pulsing the DONE pin, TPL5110 pulls DRV HIGH after the timeout, resetting the system.

| TPL5110 Pin | Pin Name | Connected To | Net | Notes |
| --- | --- | --- | --- | --- |
| Pin 1 | VDD | VCC_3V3 | VCC_3V3 | Power supply 1.8V–5.5V. |
| Pin 2 | DELAY/N_DRV | Resistor to GND (sets timeout period) | WDG_DELAY | 100kΩ → ~100s timeout. 10kΩ → ~10s. See datasheet Table 1. |
| Pin 3 | AUX_DA | PCF8574T AUX_DA / NC | AUX_CL | ⚠ In schematic, AUX_DA and AUX_CL from MPU-6050 shown connecting here. Verify intent. |
| Pin 4 | AUX_CL | PCF8574T AUX_CL / NC | — | Same note as above. |
| Pin 5 | GND | DGND | GND | Ground. |
| Pin 6 | DONE | ESP32 IO32 (WDG_FEED net) | WDG_FEED | ESP32 pulses this HIGH then LOW to reset watchdog timer. |
| Pin 7 | DRV | EN pin of ESP32 via resistor OR mosfet gate | SYS_RESET | If watchdog times out, DRV goes HIGH → triggers reset. Connect to ESP32 EN. |
| Pin 8 (if present) | EN/ONE_SHOT | VCC_3V3 (always enabled) or GPIO | EN_WDG | Enable pin. HIGH = watchdog active. Tie HIGH for always-on protection. |

Watchdog timeout is set by the resistor on DELAY/N_DRV pin. Use 100kΩ for ~100s timeout — firmware must pulse IO32 (WDG_FEED) at least once every 100 seconds. Shorter timeouts (10kΩ=10s) are safer but require more frequent feeding.

# Section 12 — PCF8574T I/O Expander (U11)

The PCF8574T adds 8 additional GPIO pins via the I2C bus. P0–P7 can be configured as inputs or outputs.

| PCF8574T Pin | Net / Signal | Connected To | Notes |
| --- | --- | --- | --- |
| VDD (Pin 16) | VCC_3V3 | AMS1117-3.3 output | Power. 2.5–6V range. |
| GND (Pin 8) | GND | DGND plane | Ground. |
| SDA (Pin 15) | I2C_SDA | I2C bus (shared with all I2C devices) | Open-drain. 4.7kΩ pull-up on bus. |
| SCL (Pin 14) | I2C_SCL | I2C bus clock from ESP32 | Open-drain. |
| INT (Pin 13) | INT_PCF | DS3231M or ESP32 GPIO (interrupt line) | Active LOW when any P pin changes state. Connect to ESP32 input pin. |
| A0 (Pin 1) | GND | GND — I2C address bit 0 = 0 | I2C address = 0x20 when A2=A1=A0=GND. |
| A1 (Pin 2) | GND | GND — I2C address bit 1 = 0 |  |
| A2 (Pin 3) | GND | GND — I2C address bit 2 = 0 |  |
| P0 (Pin 4) | P0 | GPIO expansion — LED or relay control | Output/Input as needed via I2C writes. |
| P1 (Pin 5) | P1 | GPIO expansion |  |
| P2 (Pin 6) | P2 | GPIO expansion |  |
| P3 (Pin 7) | P3 | GPIO expansion |  |
| P4 (Pin 9) | P4 | GPIO expansion |  |
| P5 (Pin 10) | P5 | GPIO expansion |  |
| P6 (Pin 11) | P6 | GPIO expansion |  |
| P7 (Pin 12) | P7 | GPIO expansion | P0–P7 all available for additional outputs (LEDs, relays, etc.) |

⚠  PCF8574T address pins A0=A1=A2=GND give address 0x20. DS3231M is 0x68, MPU-6050 will be 0x69, EEPROM is 0x50 — no conflicts with PCF8574T. Verify correct addressing in firmware.

# Section 13 — MCP100-450D Supervisor (UL2)

The MCP100-450D monitors the 3.3V rail and holds ESP32 in reset during power-up until voltage is stable.

| MCP100-450D Pin | Signal | Connected To | Notes |
| --- | --- | --- | --- |
| VDD (Pin 3) | VCC_3V3 | AMS1117-3.3 output | Monitors this voltage. Resets if it drops below 4.5V threshold. |
| GND (Pin 1) | GND | DGND | Ground reference. |
| /RESET (Pin 2) | SYS_RESET | ESP32 EN pin (Active LOW) | Open-drain output. Goes LOW when VDD < 4.5V threshold. ESP32 resets. |
| External pull-up | — | 10kΩ from SYS_RESET to VCC_3V3 | Required — MCP100 is open-drain output. Add 10kΩ pull-up. |
| Reset Button (optional) | — | Push button from SYS_RESET to GND | Manual reset capability for development. |

⚠  MCP100-450D threshold is 4.5V — it monitors VDD. If VDD = 3.3V rail, the 4.5V threshold means it will ALWAYS keep RESET asserted (3.3V < 4.5V). Use a 3.08V threshold version (MCP100-300) or switch to APX803-31 (3.08V threshold) or TPS3839G33 (3.3V threshold) which is appropriate for a 3.3V rail.

# Section 14 — SIM Card Holder (J1 — SIN_Card_Shielded)

| SIM Card Pin | Net | Connected To | Notes |
| --- | --- | --- | --- |
| Pin 1 (C1 VCC) | USIM_VCC | EC200U USIM_VDD pin | EC200U supplies 1.8V or 3V SIM power. Add 47pF to GND. |
| Pin 2 (C2 RST) | USIM_RST | EC200U USIM_RST pin | Active LOW reset. 22Ω series + 47pF to GND. |
| Pin 3 (C3 CLK) | USIM_CLK | EC200U USIM_CLK pin | Clock from EC200U to SIM. 22Ω series. |
| Pin 5 (C5 GND) | GND | DGND plane | SIM card ground. |
| Pin 6 (C6 VPP) | VPP | NC — not used in modern SIMs | Leave NC for nano-SIM. |
| Pin 7 (C7 I/O) | USIM_DATA | EC200U USIM_DATA pin | Bidirectional data. 22Ω + 47pF to GND. |
| SH (Shield) | GND | DGND — shielding shell of SIM holder | Ground the metal shell for EMI shielding. |
| Card Detect SW | USIM_DET | EC200U USIM_DET pin (or GPIO) | Closes when SIM inserted. 47kΩ pull to 1.8V. |

⚠  Missing ESD protection on SIM lines. Add PRTR5V0U2X or similar SIM TVS IC on USIM_DATA and USIM_CLK. SIM card lines are ESD-sensitive.

# Section 15 — Antenna Connections (J2, J3)

| Connector | Type | Signal | Connected To | Notes |
| --- | --- | --- | --- | --- |
| J2 | SMA_Edge_SMD | LTE_ANT (Main) | EC200U MAIN_ANT pin via R1+C2 matching | 50Ω microstrip. R1 (0Ω or small series) + C2 (100nF) for matching. Antenna: external LTE whip 700–2600MHz. |
| J3 | SMA_Edge_SMD | GNSS_ANT | EC200U ANT_GNSS pin | 50Ω coax. Active GPS patch antenna (3.3V bias via ferrite on SMA center pin). Passive patch also works. |
| GND | SMA shell | PGND | Board ground plane — SMA shell must connect to solid copper pour | Keep RF ground solid. No other traces under antenna footprint. |

⚠  No antenna bias circuit visible for GNSS active antenna on J3. If using an active GPS patch antenna, you MUST add a bias-T: ferrite bead (100nF cap in series on signal line + 3.3V through ferrite on center pin). Passive antenna does not need this.

# Section 16 — Issues Found & Required Fixes

14 issues were identified during schematic review. Issues are ranked by severity.

| # | Issue Found | Location | Severity | Recommended Fix |
| --- | --- | --- | --- | --- |
| 1 | EC200U VBAT connected to 5V rail directly — module max is 4.3V | U13 VBAT / U3 output | CRITICAL | Add AMS1117-4.2 or LP5907-4.2 LDO between 5V rail and EC200U VBAT. Powering at 5V WILL damage the module. |
| 2 | No reverse polarity protection on OBD-II 12V input (Pin 16) | J6 Pin 16 | HIGH | Add series P-MOSFET (AO3401A) or Schottky diode (SS34) between OBD Pin 16 and VIN_12V net before LM2596. |
| 3 | IO18 (SWP/SD3), IO19 (X4), IO23 (SHD/SD2) have no connections | ESP32 U1 | MEDIUM | Either assign to SPI (if SD card needed) or add 10kΩ pull-down resistors to prevent floating inputs causing spurious interrupts. |
| 4 | No 4.7kΩ pull-up resistors explicitly shown on I2C SDA/SCL lines | I2C bus | HIGH | Add explicit 4.7kΩ from I2C_SDA to VCC_3V3 and 4.7kΩ from I2C_SCL to VCC_3V3. Required for I2C bus to function. |
| 5 | DS3231M address conflict with MPU-6050 — both default to 0x68 | U8 DS3231M / U9 MPU-6050 | HIGH | Change MPU-6050 I2C address: connect AD0 pin to VCC_3V3 instead of GND → sets address to 0x69. DS3231M stays at 0x68. |
| 6 | C3 (1µF supercap) on DS3231M VBAT is too small for RTC backup | U8 DS3231M pin VBAT | MEDIUM | Replace 1µF with a 3V CR2032 coin cell or at least a 100–220µF supercapacitor for meaningful RTC backup time. |
| 7 | EC200U PWRKEY and RESET_N have no pull-up resistors | U13 PWRKEY / RESET_N | MEDIUM | Add 10kΩ pull-up from PWRKEY to 1.8V (VDD_EXT) to keep module powered. Same for RESET_N → keeps it deasserted. |
| 8 | CAN bus termination resistors (120Ω) not visible in schematic | CAN bus | MEDIUM | Add two 120Ω resistors (0805) on CANH-CANL. Include solder-bridge jumpers so termination can be disabled if device is mid-bus. |
| 9 | SIM card ESD protection not shown | SIM Card J1 | MEDIUM | Add PRTR5V0U2X (NXP) or similar dual TVS on USIM_DATA and USIM_CLK lines. Also add 22Ω series resistors on all SIM lines. |
| 10 | F8 purpose not clear — appears between LM2596 ON/OFF and VCC_5V | U3 LM2596 area | LOW | Verify F8 is a polyfuse or ferrite bead. If it's a fuse, rate it 2A for the 5V rail. If ferrite, confirm it doesn't drop too much voltage. |
| 11 | SENSOR_VP (IO36) and SENSOR_VN (IO39) are input-only, no external connections shown | ESP32 U1 | LOW | These are input-only ADC pins. Either use for battery voltage monitoring (via voltage divider from VIN_12V) or leave NC with explicit NC markers. |
| 12 | No LED current limiting resistors explicitly shown for LED1 (D1) | LED1 / D1 area | MEDIUM | Add 330Ω (0402) in series with each LED for 3.3V supply (gives ~9mA). Verify 330Ω is in schematic between GPIO and LED anode. |
| 13 | EC200U AUX_RXD/TXD and DBG_RXD/TXD left floating | U13 AUX/DBG UART | LOW | Tie AUX_RXD to 1.8V via 10kΩ (keep idle HIGH). AUX_TXD and DBG pins can be NC. Add test points for development. |
| 14 | GRF_C1 and GRF_C2 pins on EC200U have no decoupling caps shown | U13 GRF pins | HIGH | Add 100nF capacitor from each GRF_C1 and GRF_C2 pin to GND. These are RF decoupling pins — required by Quectel hardware design guide. |

✘  Fix Issues #1 (VBAT voltage), #5 (I2C address conflict), and #13 (MCP100 threshold) BEFORE ordering PCBs — these will cause the design to not function.

✔  Overall schematic structure is well-organized. All major functional blocks are present. The design covers CAN bus, GPS, 4G LTE, USB programming, RTC, IMU, I2C expansion, watchdog, and cloud connectivity.