# DriveEasy A7670C GSM UART Integration Report

This document provides a complete step-by-step professional guide to connect and configure the A7670C GSM LTE module with Raspberry Pi 4 using UART protocol.

## 1. Hardware Requirements

- Raspberry Pi 4

- A7670C GSM LTE module (UART version)

- Airtel IoT SIM card (activated with data plan)

- External 4.0V regulated power supply (minimum 2A current capability)

- 1000µF capacitor across VBAT and GND (recommended)

- Connecting wires

## 2. Power Supply Requirements (Critical)

The A7670C requires a stable 3.8V–4.2V supply with peak current up to 2A. Do NOT power the module directly from Raspberry Pi 5V pin. Use a 5V to 4V buck converter and ensure GSM GND is connected to Raspberry Pi GND.

## 3. UART Wiring Connections

Connect the module as follows:

A7670C TXD  → Raspberry Pi GPIO15 (Pin 10)

A7670C RXD  → Raspberry Pi GPIO14 (Pin 8)

A7670C GND  → Raspberry Pi GND (Pin 6)

A7670C VCC  → External 4.0V supply

Ensure TX is connected to RX and RX to TX. Common ground is mandatory.

## 4. Enable UART on Raspberry Pi

Run the following command:

sudo raspi-config

Navigate to: Interface Options → Serial Port

Set 'Login shell over serial' to NO

Set 'Serial hardware enabled' to YES

Reboot the Raspberry Pi using:

sudo reboot

## 5. Disable Bluetooth UART Conflict (Recommended)

Edit boot configuration file:

sudo nano /boot/config.txt

Add the following lines at the bottom:

dtoverlay=disable-bt

enable_uart=1

Save and reboot: sudo reboot

## 6. Verify Serial Port

After reboot, check UART device:

ls -l /dev/serial0

It should point to ttyAMA0.

## 7. Install Serial Terminal Software

Install screen utility:

sudo apt update

sudo apt install screen

## 8. Open Serial Communication

Use the following command:

sudo screen /dev/ttyAMA0 115200

If no response, try 9600 baud rate:

sudo screen /dev/ttyAMA0 9600

## 9. Test AT Communication

Type the following command inside screen:

AT

Expected response: OK

## 10. SIM and Network Check Commands

Check SIM status: AT+CPIN?

Check signal strength: AT+CSQ

Check network registration: AT+CREG?

Attach to packet service: AT+CGATT=1

Set APN (Airtel example): AT+CGDCONT=1,"IP","airtelgprs.com"

Activate PDP: AT+CGACT=1,1

Open network: AT+NETOPEN

Get IP address: AT+IPADDR

## 11. Test HTTP Connectivity

Initialize HTTP: AT+HTTPINIT

Set CID: AT+HTTPPARA="CID",1

Set URL: AT+HTTPPARA="URL","http://httpbin.org/get"

Start GET request: AT+HTTPACTION=0

Read response: AT+HTTPREAD

## 12. Troubleshooting Guide

- If garbage characters appear, check baud rate.

- Ensure common ground between Pi and GSM.

- Verify TX/RX are not swapped.

- Ensure stable 4V power supply.

- Disable serial console and Bluetooth UART.

End of Report