# GSM (A7670C) Internet Setup Guide
Raspberry Pi + UART + PPP Method

This document provides a complete step-by-step procedure to:
1) Verify GSM Internet connectivity using AT commands.
2) Configure PPP over UART to provide internet access inside Raspberry Pi Linux.
sudo reboot

## SECTION 1: Hardware Requirements

• Raspberry Pi 4

• A7670C GSM/LTE Module

• FTDI (CP210x) USB to UART Converter

• Airtel IoT SIM

• Stable 4.0V–4.2V Power Supply (Minimum 2A current)

• LTE Antenna connected to GSM module

## SECTION 2: Wiring Connections (UART)

FTDI TX  →  GSM RX

FTDI RX  →  GSM TX

FTDI GND →  GSM GND

⚠ Ensure FTDI logic level is set to 3.3V.

⚠ Ensure power supply can provide at least 2A peak current.

## SECTION 3: Verify GSM Connectivity (AT Commands)

1. Connect GSM via FTDI to Raspberry Pi.

2. Check serial port:

ls /dev/ttyUSB*

3. Open serial terminal:

sudo screen /dev/ttyUSB0 115200

4. Test modem response:

AT

5. Check SIM status:

AT+CPIN?

6. Check signal strength:

AT+CSQ

7. Check network registration:

AT+CREG?

8. Attach to network:

AT+CGATT=1

9. Set APN (Airtel IoT):

AT+CGDCONT=1,"IP","airtel.iot"

10. Open data network:

AT+NETOPEN

11. Check assigned IP:

AT+IPADDR

If an IP like 10.x.x.x appears, GSM internet is active inside the modem.

Exit screen using: CTRL + A, then K, then Y.

## SECTION 4: Disable WiFi Before PPP

sudo ifconfig wlan0 down

## SECTION 5: Configure PPP Over UART

1. Create PPP peer file:

sudo nano /etc/ppp/peers/a7670

Paste the following:

/dev/ttyUSB0 115200
connect "/usr/sbin/chat -v -f /etc/ppp/chat-a7670"
noauth
defaultroute
usepeerdns
lock
debug

2. Create chat script file:

sudo nano /etc/ppp/chat-a7670

Paste the following:

ABORT "ERROR"
ABORT "NO CARRIER"
"" AT
OK ATE0
OK AT+CGDCONT=1,"IP","airtel.iot"
OK ATD*99***1#
CONNECT ""

## SECTION 6: Start PPP Connection

Stop any existing PPP session:

sudo poff

Start PPP:

sudo pon a7670

Wait 15–20 seconds, then verify:

ifconfig

If successful, interface 'ppp0' will appear.

Test internet:

ping 8.8.8.8

## SECTION 7: Debugging PPP

If PPP fails, check logs:

sudo plog

Ensure modem is not rebooting and power supply is stable.

## Important Notes

• UART speed limits bandwidth (~8–10 KB/sec).

• Suitable for telemetry, MQTT, small HTTP requests.

• Not recommended for video streaming (WebRTC).

• For high bandwidth applications, use USB LTE interface instead of UART.