# Raspberry Pi 4 + A7670C GSM Internet Setup Report

Generated on: 27-02-2026 05:47:52

Project: GSM Internet Access via UART (Without WiFi)

Hardware: Raspberry Pi 4 Model B + SIM A7670C GSM Module

------------------------------------------------------------

## 1. Hardware Requirements

- Raspberry Pi 4 Model B

- A7670C GSM Module

- 4.0V–4.2V External Power Supply (Minimum 2A Current)

- GSM Antenna

- HDMI Monitor

- USB Keyboard & Mouse

- Jumper Wires

## 2. Hardware Connections (UART Wiring)

Connect Raspberry Pi GPIO to A7670C as follows:

Pin 8 (GPIO14 TXD)  →  A7670C RX

Pin 10 (GPIO15 RXD) →  A7670C TX

Pin 6 (GND)         →  A7670C GND

External 4.1V Supply → A7670C VBAT

NOTE: Do NOT power GSM module from Raspberry Pi 5V or 3.3V.

## 3. Disable WiFi on Raspberry Pi

Open terminal and run:

nmcli device status

sudo nmcli device disconnect wlan0

sudo rfkill block wifi

Verify:

nmcli device status

## 4. Enable UART on Raspberry Pi

Run:

sudo raspi-config

Navigate to: Interface Options → Serial Port

Select:

Login shell over serial? → NO

Enable serial hardware? → YES

Reboot:

sudo reboot

## 5. Install Minicom for AT Command Testing

sudo apt update

sudo apt install minicom -y

## 6. Test GSM Module via AT Commands

Open serial communication:

sudo minicom -D /dev/serial0 -b 115200

Inside minicom, type:

AT

Expected Response: OK

Check SIM & Network:

AT+CPIN?

AT+CSQ

AT+CREG?

AT+CGATT?

Expected:

CPIN → READY

CSQ → Greater than 10

CREG → 0,1

CGATT → 1

## 7. Configure APN (Airtel Example)

Normal Airtel SIM:

AT+CGDCONT=1,"IP","airtelgprs.com"

Airtel IoT SIM:

AT+CGDCONT=1,"IP","airtel.iot"

Activate PDP Context:

AT+CGACT=1,1

Exit Minicom: CTRL + A then X

## 8. Install PPP for Internet Access

sudo apt install ppp -y

## 9. Create PPP Configuration File

Create peer file:

sudo nano /etc/ppp/peers/a7670

Paste below content:

/dev/serial0

115200

crtscts

defaultroute

usepeerdns

noauth

persist

connect "/usr/sbin/chat -v -f /etc/chatscripts/a7670-chat"

## 10. Create Chat Script

sudo nano /etc/chatscripts/a7670-chat

Paste below content:

ABORT "ERROR"

ABORT "NO CARRIER"

ABORT "BUSY"

ABORT "NO ANSWER"

"" AT

OK AT+CGDCONT=1,"IP","airtelgprs.com"

OK ATD*99#

CONNECT ""

## 11. Start GSM Internet

sudo pon a7670

Check Interface:

ifconfig ppp0

Test Internet:

ping 8.8.8.8

## 12. Stop GSM Internet

sudo poff a7670

------------------------------------------------------------

End of Report